Network Device SDK (Android)

User Manual

Manual Version: V2.6.0

Disclaimer

No part of this manual may be copied, reproduced, translated or distributed in any form or by any means without prior consent in writing from Zhejiang Uniview Technologies Co., Ltd (referred to as Uniview or us hereafter).

Due to such reasons as product version upgrade or regulatory requirement of relevant regions, this manual will be periodically updated. Uniview reserves the right to change any information in this manual without any prior notice or indication. This manual is only for informational purpose, and all statements, information, and recommendations in this manual are presented without warranty.

# Contents

1 Contents	2

2 Preface	6

3 Introduction	6

3.1 Overview	6

3.2 System Requirements	6

4 Version Updates	7

4.1 Release History	7

4.2 Revision Description	8

5 Programming Environment Setup	9

5.1 JNI Development Environment Setup	9

5.1.1 JNI Overview	9

5.1.2 Development Environment Setup	9

5.2 Android SDK Development and Compilation Environment on Windows	10

6 Programming Guide	10

6.1 Programming Guide	10

6.2 Main Process of Calling Android SDK	11

7 APIs	14

7.1 Macro Definitions, Enumeration Values, and Struct Definitions	14

7.2 SDK Initialization	14

7.2.1 NETDEV_Init	14

7.2.2 NETDEV_Cleanup	14

7.3 User Login and Logout	15

7.3.1 NETDEV_Login	15

7.3.2 NETDEV_Login_V30	15

7.3.3 NETDEV_Logout	16

7.4 Query Channel Capability Sets	16

7.4.1 NETDEV_QueryVideoChlDetailList	16

7.4.2 NETDEV_QueryVideoChlDetailListEx	17

7.5 Live View	17

7.5.1 NETDEV_GetStreamUrl	17

7.5.2 NETDEV_FastRealPlayByUrl	18

7.5.3 NETDEV_RealPlay	19

7.5.4 NETDEV_StopRealPlay	19

7.5.5 NETDEV_CapturePicture	20

7.5.6 NETDEV_SaveRealData	20

7.5.7 NETDEV_StopSaveRealData	22

7.5.8 NETDEV_OpenSound	22

7.5.9 NETDEV_CloseSound	22

7.6 Playback	23

7.6.1 NETDEV_FindFile	23

7.6.2 NETDEV_FindNextFile	23

7.6.3 NETDEV_FindClose	24

7.6.4 NETDEV_GetReplayUrl	24

7.6.5 NETDEV_FastPlayBackByUrl	25

7.6.6 NETDEV_PlayBackByTime	26

7.6.7 NETDEV_PlayBackControl	26

7.6.8 NETDEV_StopPlayBack	27

7.6.9 NETDEV_QuickSearch	27

7.7 Play Settings	28

7.7.1 NETDEV_SetPictureFluency	28

7.8 Stream Callback	29

7.8.1 NETDEV_SetPlayDecodeAudioCB	29

7.9 Recording Download	29

7.9.1 NETDEV_DownloadFile	29

7.9.2 NETDEV_StopDownloadFile	30

7.10 Non-Live View Capture	30

7.10.1 NETDEV_CaptureNoPreview	30

7.11 Display Service	31

7.11.1 Initialize	31

7.11.2 setRendererViewport	32

7.11.3 initializeRenderer	32

7.11.4 rendererRender	33

7.11.5 Scale	33

7.12 Time Settings	34

7.12.1 NETDEV_GetSystemTimeCfg	34

7.12.2 NETDEV_SetSystemTimeCfg	34

7.13 PTZ Control	35

7.13.1 NETDEV_PTZControl	35

7.13.2 NETDEV_PTZControl_Other	36

7.13.3 NETDEV_GetPTZPresetList	38

7.13.4 NETDEV_GetPTZPresetListEx	39

7.13.5 NETDEV_PTZPreset	39

7.13.6 NETDEV_PTZPreset_Other	40

7.13.7 NETDEV_PTZAbsoluteMove	41

7.13.8 NETDEV_PTZGetStatus	41

7.14 Two-way Audio	42

7.14.1 NETDEV_StartInputVoiceSrv	42

7.14.2 NETDEV_Android_StartInputVoiceSrv	43

7.14.3 NETDEV_InputVoiceData	43

7.14.4 NETDEV_StopInputVoiceSrv	44

7.14.5 NETDEV_Android_StopInputVoiceSrv	44

7.15 Organization Info	45

7.15.1 NETDEV_FindOrgInfoList	45

7.15.2 NETDEV_FindNextOrgInfo	45

7.15.3 NETDEV_FindCloseOrgInfo	46

7.15.4 NETDEV_FindOrgChnList	46

7.15.5 NETDEV_FindNextOrgChn	47

7.15.6 NETDEV_FindCloseOrgChn	47

7.16 Device Info	48

7.16.1 NETDEV_GetDeviceInfo	48

7.16.2 NETDEV_FindDevList	48

7.16.3 NETDEV_FindNextDevInfo	49

7.16.4 NETDEV_FindCloseDevInfo	50

7.16.5 NETDEV_FindDevChnList	50

7.16.6 NETDEV_FindNextDevChn	51

7.16.7 NETDEV_FindCloseDevChn	51

7.17 Configuration Info	52

7.17.1 NETDEV_GetDevConfig	52

7.17.2 NETDEV_SetDevConfig	53

7.18 Register Callback	54

7.18.1 NETDEV_SetAlarmCallBackV2	54

7.18.2 NETDEV_Android_SetAlarmCallBack	54

7.18.3 NETDEV_SetAlarmCallBack_V30	55

7.18.4 NETDEV_Android_SetAlarmCallBack_V30	55

7.18.5 NETDEV_SetExceptionCallBack	56

7.18.6 NETDEV_Android_SetExceptionCallBack	56

7.19 Alarm Subscription	57

7.19.1 NETDEV_SubscibeLapiAlarm	57

7.19.2 NETDEV_UnSubLapiAlarm	58

7.20 Query People Flow	58

7.20.1 NETDEV_StartMultiTrafficStatistic	58

7.20.2 NETDEV_GetTrafficStatisticProgress	59

7.20.3 NETDEV_StopTrafficStatistic	59

7.20.4 NETDEV_FindTrafficStatisticInfoList	60

7.20.5 NETDEV_FindNextTrafficStatisticInfo	60

7.20.6 NETDEV_FindCloseTrafficStatisticInfo	61

7.20.7 NETDEV_SetPassengerFlowStatisticCallBack	61

7.21 Device Discovery	62

7.21.1 NETDEV_Discovery	62

7.21.2 NETDEV_SetDiscoveryCallBack	62

7.21.3 NETDEV_ModifyDeviceAddr	63

7.22 Get Error Code	63

7.22.1 NETDEV_GetLastError	63

7.22.2 Error Code List	63

8 Data Structure	74

8.1 NETDEV_DEVICE_INFO_S	74

8.2 NETDEV_FILECOND_S	74

8.3 NETDEV_FINDDATA_S	74

8.4 NETDEV_PLAYBACKCONTROL_S	75

8.5 NETDEV_PREVIEWINFO_S	75

8.6 NETDEV_VIDEO_CHL_DETAIL_INFO_S	76

8.7 NETDEV_VIDEO_CHL_DETAIL_INFO_EX_S	76

8.8 NETDEV_TIME_S	77

8.9 NETDEV_MONTH_INFO_S	78

8.10 NETDEV_MONTH_STATUS_S	78

8.11 NETDEV_DEVICE_BASICINFO_S	78

8.12 NETDEV_PLAYBACKCOND_S	78

8.13 NETDEV_ALARMINFO_S	80

8.14 NETDEV_AUDIO_SAMPLE_PARAM_S	80

8.15 NETDEV_PTZ_ABSOLUTE_MOVE_S	81

8.16 NETDEV_PTZ_ALLPRETSET_S	81

8.17 NETDEV_PTZ_PRESET_S	81

8.18 NETDEV_PTZ_STATUS_S	82

8.19 NETDEV_DEVICE_LOGIN_INFO_S	82

8.20 NETDEV_SELOG_INFO_S	83

8.21 NETDEV_ORG_FIND_COND_S	83

8.22 NETDEV_ORG_INFO_S	84

8.23 NETDEV_ORG_CHN_INFO_S	85

8.24 NETDEV_DEV_BASIC_INFO_S	85

8.25 NETDEV_DEV_CHN_ENCODE_INFO_S	88

8.26 NETDEV_MULTI_TRAFFIC_STATISTICS_COND_S	89

8.27 NETDEV_TRAFFIC_STATISTICS_INFO_S	90

8.28 AlarmCallBack_PF	90

8.29 AlarmMessCallBackV30	97

8.30 ExceptionCallBack_PF	106

8.31 NETDEV_DISCOVERY_DEVINFO_S	107

8.32 NETDEV_DEV_ADDR_INFO_S	107

# Preface

Thank you for choosing our product. Should you have any questions, please do not hesitate to contact us.

No part of this manual may be reproduced or transmitted in any form or by any means without written authorization. The information in this manual is subject to change without notice. Every effort has been made in the preparation of this manual to ensure accuracy of the contents, but no statements, information, and recommendations in this manual should constitute the warranty of any kind, express or implied.

# Introduction

## Overview

The Android SDK provides a set of APIs developed based on private network communication protocols, allowing developers to develop software with the ability to remotely access and control IP devices, such as IPC and NVR.

The APIs are categorized as follows:

•	Device management

•	Live view and playback

The Android SDK is provided as a dynamic link library. The JNI layer that communicates with the NetDevSDK library is provided in the form of source code.

This manual details the functionality and usage of functions included in the Android SDK and their relationship.

## System Requirements

Mobile devices:

•	Tested system: Android 6.0/N

Windows:

•	32-bit and 64-bit SDK

•	Tested system: Windows 7

# Version Updates

## Release History

| Manual Version | Device Version | Release Date | Remarks |
| --- | --- | --- | --- |
| V1.0.0 |  | 2016.11 | Initial release |
| V1.8.0 |  | 2018.07 | 2 expansion interfaces: Added split screen function, expansion interface NETDEV_RealPlay NETDEV_StopRealPlay 14 new interfaces: PTZ control: NETDEV_PTZPreset_Other NETDEV_PTZPreset NETDEV_GetPTZPresetList NETDEV_PTZControl Recording download: NETDEV_DownloadFile NETDEV_StopDownloadFile Add/delete cloud devices: NETDEV_AddCloudDevice NETDEV_DeleteCloudDevice Capture during live view: NETDEV_CapturePicture Query recordings by month: NETDEV_QuickSearch System time: NETDEV_GetSystemTimeCfg Get device info: NETDEV_GetDeviceInfo Local recording: NETDEV_SaveRealData NETDEV_StopSaveRealData Update error code list |
| V2.2.0 |  | 2019.08 | Supports 64-bit Android |
| V2.3.0 |  | 2019.10 | 22 new interfaces: Register alarm callback: NETDEV_SetAlarmCallBack_V30 NETDEV_Android_SetAlarmCallBack_V30 PTZ control: NETDEV_GetPTZPresetListEx NETDEV_PTZControl_Other NETDEV_PTZAbsoluteMove NETDEV_PTZGetStatus Get device configuration info: NETDEV_GetDevConfig Set device configuration info: NETDEV_SetDevConfig Login: NETDEV_Login_V30 Query channel capability set: NETDEV_QueryVideoChlDetailListEx Query organization list: NETDEV_FindOrgInfoList NETDEV_FindNextOrgInfo NETDEV_FindCloseOrgInfo Query channel list by organization ID: NETDEV_FindOrgChnList NETDEV_FindNextOrgChn NETDEV_FindCloseOrgChn Query device list by device type: NETDEV_FindDevList NETDEV_FindNextDevInfo NETDEV_FindCloseDevInfo Query channel list by device ID or channel type: NETDEV_FindDevChnList NETDEV_FindNextDevChn NETDEV_FindCloseDevChn Update error code list |
| V2.5.0 |  | 2020.07 | Start multi-channel people flow statistics query: NETDEV_StartMultiTrafficStatistic Get people flow counting progress: NETDEV_GetTrafficStatisticProgress Stop people flow statistics query: NETDEV_StopTrafficStatistic Get people flow statistics list: NETDEV_FindTrafficStatisticInfoList NETDEV_FindNextTrafficStatisticInfo NETDEV_FindCloseTrafficStatisticInfo Open sound: NETDEV_OpenSound Register stream callback function: NETDEV_SetPlayDecodeAudioCB |
| V2.6.0 |  | 2021.12 | Added device discovery function: NETDEV_Discovery NETDEV_SetDiscoveryCallBack NETDEV_ModifyDeviceAddr |

## Revision Description

##### V1.0.0

Initial release.

##### V2.3.0

Supports VMS.

##### V2.5.0

Supports people flow query, turn on/off sound, bug fixes.

##### V2.6.0

Supports device discovery.

# Programming Environment Setup

## JNI Development Environment Setup

### JNI Overview

JNI (Java Native Interface) provides several APIs to implement communication between Java and other programming languages (mainly C&C++). Since Java 1.1, the JNI standard has become part of the Java platform, which allows Java code to interact with code written in other programming languages. JNI was primarily designed for natively compiled languages, especially C and C++, but it does not prevent you from using other programming languages as long as the calling conventions are supported. Using Java to interact with natively compiled code usually results in a loss of platform portability. However, there are cases where this is acceptable or even required. Android only allows Java-embedded native C/C++ code-use native code via JNI instead of pure C/C++ code. Therefore JNI is very important for Android underlying development.

### Development Environment Setup

#### NDK Environment Setup

NDK development on Windows:

1. Download the correct NDK version for Windows.

2. Extract the NDK package. The extraction path cannot contain spaces, otherwise an error will occur during compilation.

3. Configure environment variables. Go to Control Panel > System and Security > System > Advanced system settings > Environment Variables, select Path variable under System variables and click Edit. If there is no Path variable, click New to create one. Add the path of the NDK folder at the end of the variable value, and use “;” to separate the path from the preceding variable value.

As shown above, the path of ndk-build.cmd in NDK is D:\software\android-ndk-r10d, then add ;D:\software\android-ndk-r10d at the end of the variable value. If the CLI window is opened before the above configuration, the configuration takes effect the next time you open the CLI window.

#### JDK Environment Setup

Users can find JDK environment setup tutorials on the Internet.

Purpose: Use a Java package to create a header file, and then use the header file to implement the corresponding interface via c/c++.

#### Android Compilation Environment Setup

Note: This manual uses Android Studio as an example to describe Android development.

Users can find Android compilation environment setup tutorials on the Internet.

And users need to download Android SDK for Android Studio by themselves.

Purpose: Create Android projects and compile.

## Android SDK Development and Compilation Environment on Windows

Obtain Android SDK for Windows (AndroidSDK.zip) and then extract it to the desired directory, for example, E:\ AndroidSDK.

Now the directory contains these folders:

android: Contains the dynamic library (.so) and static library (.a) for Android SDK development.

java: Contains the JNI source code for Android SDK development to connect the dynamic library and the Android app.

netdevsdk Demo: Contains the demo source code developed with Java.

# Programming Guide

## Programming Guide

SDK Functional Interface File

NetDevSDK.java: Provides definitions of macros, enumerations and functional interfaces for the Java layer.

NetDEVSDK.h: Provides definitions of macros, enumerations, data structures, and functional interfaces for the JNI layer.

Programming Instructions

1. String format

UTF-8 code is required for non-English characters.

2. Time description

C++: Seconds since Unix epoch (January 1 1970 00:00:00 GMT) till now.

Java: Milliseconds since Unix epoch (January 1 1970 00:00:00 GMT) till now.

3. Get error code

When calling a function in the SDK, FALSE will be returned if an exception occurs inside the function (such as live view failure). Call NETDEV_GetLastError() to get the error code. See the error code list for details.

## Main Process of Calling Android SDK

Main Process of Calling Android SDK APIs

Process of Calling Realplay Module APIs

Process of Calling Playback Module APIs

As shown above, the Android SDK can be divided into multiple modules by function. The following four processes are essential to implement the functions of each module: initialize SDK, user login, user logout, and release SDK resources.

•	Initialize SDK (NETDEV_Init): Initialize the whole SDK, pre-allocate memory and so on.

•	User Login (NETDEV_Login_V30): Implement user login function. After login succeeded, the returned user ID will be the unique identifier for other operations.

•	Cloud Account Login (NETDEV_LoginCloud): Implement cloud account login function. After login succeeded, the returned cloud account ID will be the unique identifier for the cloud account.

•	Query Cloud Account Device List (NETDEV_FindCloudDevList): Get device list of a cloud account. After query succeeded, a query handle will be returned to call NETDEV_FindNextCloudDevInfo to get device info under the cloud account one by one, and call NETDEV_FindCloseCloudDevList to close the function to release resources after query is finished.

•	Cloud Device Login (NETDEV_LoginCloudDevice_V30): Implement cloud device login function. After login succeeded, the returned user ID will be the unique identifier for other operations.

•	Query Device List by Device Type (NETDEV_FindDevList): Get logged-in device list. After query succeeded, a query handle will be returned to call NETDEV_FindNextDevInfo to get logged-in device info one by one, and call NETDEV_FindCloseDevList to close the function to release resources after query is finished.

•	Query Channel List by Device ID or Channel Type (NETDEV_FindDevChnList): Get channel list of a device. After query succeeded, a query handle will be returned to call NETDEV_FindNextDevChn to get channel info one by one, and call NETDEV_FindCloseDevChn to close the function to release resources after query is finished.

•	Query Channel Capability Set (NETDEV_QueryVideoChlDetailList): NULL means failure, other values mean channel capability sets.

•	Get Device Info (NETDEV_GetDeviceInfo): TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

•	Live View Module: Get live streams from front-end devices, decode and display. Supports obtaining live view data.

•	Start Live View (NETDEV_RealPlay): 0 means failure, other values are used as the handle of functions such as NETDEV_StopRealPlay.

•	Live View Snapshot (NETDEV_CapturePicture): TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

•	Local Recording (NETDEV_SaveRealData): TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

•	Stop Local Recording (NETDEV_StopSaveRealData): TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

•	Stop Live View (NETDEV_StopRealPlay): TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

•	Playback Module: Query, remotely play, and decode recordings from the past 12 hours.

•	Query Recordings by File Type or Time (NETDEV_FindFile): Query recordings of a device by file type or time. After query succeeded, a query handle will be returned to call NETDEV_FindNextFile to get recordings one by one, and call NETDEV_FindClose to close the function to release resources after query is finished.

•  Playback by Time (NETDEV_PlayBackByTime): TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

•  Control Playback Status (NETDEV_PlayBackControl): TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

•  Stop Playback (NETDEV_StopPlayBack): TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

•  User Logout (NETDEV_Logout): User logs out of the system.

•  Release SDK Resources (NETDEV_ Cleanup): Call this function to release SDK resources before user logout.

# APIs

## Macro Definitions, Enumerations, and Struct Definitions

See the AndroidDemo\netdevsdkDemo\src\com\sdk folder for details. In this folder, NetDEVSDK.java contains function declarations, macro definitions and enumerations, and the rest are struct definitions.

## SDK Initialization

### NETDEV_Init

Initialize SDK.

int NETDEV_Init(

);

Parameter

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_Cleanup

Release SDK resources.

int NETDEV_Cleanup(

);

Parameter

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## User Login and Logout

### NETDEV_Login

Device login.

long NETDEV_Login(

String                    DevIP,

Int                       DevPort,

String                    Username,

String                    Password,

NETDEV_DEVICE_INFO_S    oDeviceInfo);

Parameter

DevIP

[IN] Device IP

DevPort

[IN] Device port

Username

[IN] Username

Password

[IN] Password

oDeviceInfo

[OUT] Device info struct pointer

Return Value

0 means failure, other values mean user ID. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_Login_V30

Device login.

long NETDEV_Login_V30(

NETDEV_DEVICE_LOGIN_INFO_S stDevLoginInfo,

NETDEV_SELOG_INFO_S stSELogInfo

);

Parameter

stDevLoginInfo

[IN] Login info

stSELogInfo

[OUT] Secure login info

Return Value

User ID

Remarks

### NETDEV_Logout

User logout.

Int NETDEV_Logout(

long    lpUserID

);

Parameter

lpUserID

[IN] User login ID

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Query Channel Capability Sets

### NETDEV_QueryVideoChlDetailList

Query channel capability sets.

ArrayList<NETDEV_VIDEO_CHL_DETAIL_INFO_S> NETDEV_QueryVideoChlDetailList(

long       lpUserID,

int        dwChlCount

);

Parameter

lpUserID

[IN] User login ID

dwChlCount

[INOUT] Number of channels

Return Value

NULL means failure, other values mean channel capability set list.

Remarks

### NETDEV_QueryVideoChlDetailListEx

Query channel capability sets.

int NETDEV_QueryVideoChlDetailListEx(

long iUserID,

int pdwChlCount,

ArrayList<NETDEV_VIDEO_CHL_DETAIL_INFO_EX_S> stVideoChlList

);

Parameter

lpUserID

[IN] User login ID

dwChlCount

[INOUT] Number of channels

stVideoChlList

[OUT] Channel capability set list

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Live View

### NETDEV_GetStreamUrl

Get live view stream URL.

String NETDEV_GetStreamUrl (

long lpUserID,

int dwChannelID,

int dwStreamType

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

dwStreamType

[IN] Stream type

Return Value

NULL means failure, other values are used as the handle of functions such as NETDEV_FastRealPlayByUrl.

Remarks

dwStreamType

| Value | Description |
| --- | --- |
| 0 | Main stream |
| 1 | Sub stream |
| 2 | Third stream |

### NETDEV_FastRealPlayByUrl

Start live view by URL.

long NETDEV_FastRealPlayByUrl (

long lpUserID,

String StreamUrl,

NETDEV_PREVIEWINFO_S oPreviewInfo,

int dwWinIndex

);

Parameter

lpUserID

[IN] User login ID

StreamUrl

[IN] Stream URL

oPreviewInfo

[IN] Live view parameter

dwWinIndex

[IN] Window index

Return Value

0 means failure, other values are used as the handle of functions such as NETDEV_StopRealPlay.

Remarks

### NETDEV_RealPlay

Start live view.

int NETDEV_RealPlay(

long lpUserID,

NETDEV_PREVIEWINFO_S  oPreviewInfo,

int dwWinIndex

);

Parameter

lpUserID

[IN] User login ID

oPreviewInfo

[IN] Live view parameter

dwWinIndex

[IN] Window index

Return Value

0 means failure, other values are used as the handle of functions such as NETDEV_StopRealPlay.

Remarks

### NETDEV_StopRealPlay

Stop live view.

int NETDEV_StopRealPlay(

long lpPlayID,

int dwWinIndex

);

Parameter

lpPlayID

[IN] Live view ID

dwWinIndex

[IN] Window index

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

Stop live view started by NETDEV_RealPlay.

### NETDEV_CapturePicture

Capture in live view.

Int NETDEV_CapturePicture(long lpPlayID,

String pszFileName,

int dwCaptureMode

);

Parameter

lpPlayID

[IN] Live view ID

pszFileName

[IN] Path to the saved image (including filename)

dwCaptureMode

[IN] Image format

| Value | Description |
| --- | --- |
| 0 | BMP image |
| 1 | JPG image |

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_SaveRealData

Local recording.

int NETDEV_SaveRealData(

long lpPlayID,

String pszFileName,

int dwFormat

);

Parameter

lpPlayID

[IN] Live view ID

pszFileName

[IN] Filename

dwFormat

[IN] File format

| Value | Description |
| --- | --- |
| 0 | MP4 (audio+video), filename is specified by interface |
| 1 | TS (audio+video), filename is specified by interface |
| 2 | MP4 (audio+video), filename is automatically generated according to the system time |
| 3 | TS (audio+video), filename is automatically generated according to the system time |
| 4 | TS (audio only), filename is specified by interface |
| 5 | MP4 (audio only), filename is specified by interface |
| 6 | TS (audio only), filename is automatically generated according to the system time |
| 7 | MP4 (audio only), filename is automatically generated according to the system time |
| 8 | MP4 (audio+video), filename is automatically generated according to the start and end time of the recording |
| 9 | TS (audio+video), filename is automatically generated according to the start and end time of the recording |
| 10 | MP4 (audio only), filename is automatically generated according to the start and end time of the recording |
| 11 | TS (audio only), filename is automatically generated according to the start and end time of the recording |
| 12 | MP4 (video only), filename is automatically generated according to the start and end time of the recording |
| 13 | TS (video only), filename is automatically generated according to the start and end time of the recording |
| 14 | AVI (audio+video), filename is specified by interface |
| 15 | AVI (audio+video), filename is automatically generated according to the system time |
| 16 | AVI (audio only), filename is specified by interface |
| 17 | AVI (audio only), filename is automatically generated according to the system time |
| 18 | AVI (audio+video), filename is automatically generated according to the start and end time of the recording |
| 19 | AVI (audio only), filename is automatically generated according to the start and end time of the recording |
| 20 | AVI (video only), filename is automatically generated according to the start and end time of the recording |

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_StopSaveRealData

Stop local recording.

int NETDEV_StopSaveRealData(

long lpPlayID

);

Parameter

lpPlayID

[IN] Live view ID

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_OpenSound

Open sound.

int NETDEV_OpenSound(

long lpPlayHandle

);

Parameter

lpPlayHandle

[IN] Live view handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_CloseSound

Close sound.

int NETDEV_CloseSound(

long lpPlayHandle

);

Parameter

lpPlayHandle

[IN] Live view handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Playback

### NETDEV_FindFile

Query recording files by file type and time.

long NETDEV_FindFile(

long                 lpUserID,

NETDEV_FILECOND_S   pstFindCond

);

Parameter

lpUserID

[IN] User login ID

pstFindCond

[IN] Recording query criteria

Return Value

Recording query service ID. 0 means failure, other values are used as parameters of functions such as NETDEV_FindClose. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

NETDEV_FindClose must be called after a recording query to prevent memory leaks.

### NETDEV_FindNextFile

Get recording file info one by one.

int NETDEV_FindNextFile(

long    lpFindHandle,

NETDEV_FINDDATA_S       pstFindData

);

Parameter

lpFindHandle

[IN] File query handle

pstFindData

[OUT] Pointer to the saved file info

Return Value

NETDEV_E_SUCCES means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

Failure indicates the query ended.

### NETDEV_FindClose

Close file query and release the resource.

int NETDEV_FindClose(

long lpFindHandle

);

Parameter

lpFindHandle

[IN] File query handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_GetReplayUrl

Get playback URL.

String NETDEV_GetReplayUrl(

long lpUserID,

int dwChannelID,

int dwStreamType

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

dwStreamType

[IN] Stream type

Return Value

NULL means failure, other values are used as the handle of functions such as NETDEV_FastPlayBackByUrl.

Remarks

dwStreamType

| Value | Description |
| --- | --- |
| 0 | Main stream |
| 1 | Sub stream |
| 2 | Third stream |

### NETDEV_FastPlayBackByUrl

Fast playback by URL.

long NETDEV_FastPlayBackByUrl(

long lpUserID,

String StreamUrl,

NETDEV_PLAYBACKCOND_S pstPlayBackInfo,

int dwWinIndex

);

Parameter

lpUserID

[IN] User login ID

StreamUrl

[IN] Channel ID

pstPlayBackInfo

[IN] Playback by URL struct

dwWinIndex

[IN] Window index

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_PlayBackByTime

Playback by time.

int NETDEV_PlayBackByTime(

long                        lpUserID,

NETDEV_PLAYBACKCOND_S   pstPlayBackInfo,

int dwWinIndex

);

Parameter

lpUserID

[IN] User login ID

pstPlayBackInfo

[IN] Pointer to the playback by time struct

dwWinIndex

[IN] Window index

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_PlayBackControl

Control playback status.

int NETDEV_PlayBackControl(

long                          lpPlayHandle,

int                           dwControlCode,

NETDEV_PLAYBACKCONTROL_S    lpBuffer

);

Parameter

lpPlayHandle

[IN] Live view/playback handle

dwControlCode

[IN] Command used to control playback status. See NETDEV_VOD_PLAY_CTRL_E for reference.

lpBuffer

[INOUT] Pointer to input/output parameters. See NETDEV_VOD_PLAY_STATUS_E for playback speed. The data type of playback time is int.

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

Set pdwBuffer to NULL when playback starts, pauses or resumes.

### NETDEV_StopPlayBack

Stop playback.

int NETDEV_StopPlayBack(

long   lpPlayHandle,

int 	 dwWinIndex

);

Parameter

lpPlayHandle

[IN] Playback handle

dwWinIndex

[IN] Window index

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_QuickSearch

Query recordings by month.

int[] NETDEV_QuickSearch(

long lUserID,

int dwChannelID,

NETDEV_MONTH_INFO_S oMonthInfo,

NETDEV_MONTH_STATUS_S oMonthStatus

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

oMonthInfo

[IN] Month info

oMonthStatus

[OUT] Recording status of the month

Return Value

NULL means failure, other values mean success. Call NETDEV_GetLastError() to get the error code. See Error Code List for details. The recording status list of the month is returned when the value is not NULL.

Remarks

## Play Settings

### NETDEV_SetPictureFluency

Set fluency priority.

int NETDEV_SetPictureFluency(

long lpUserID,

int dwFluency

);

Parameter

lpUserID

[IN] Live view/playback handle

dwFluency

[IN] Fluency priority

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Stream Callback

### NETDEV_SetPlayDecodeAudioCB

Register stream callback function: decoded audio stream data.

int NETDEV_SetPlayDecodeAudioCB(

long lpUserID,

String strPlayDecodeAudioCallBack,

int bContinue,

long lUserData

);

Parameter

lpUserID

[IN] Live view/playback handle

cbPlayDecodeAudioCallBack

[IN] Data callback function

bContinue

[IN] Whether to continue playing

lpUserData

[IN] User data

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Recording Download

### NETDEV_DownloadFile

Download recordings.

int NETDEV_DownloadFile(

long lpUserID,

NETDEV_PLAYBACKCOND_S lpBuffer,

String szSaveFileName,

int dwFormat

);

Parameter

lpUserID

[IN] User login ID

lpBuffer

[IN] Download info struct parameter

szSaveFileName

[IN] Path to the saved recording file (absolute path including filename)

dwFormat

[IN] Recording file format

Return Value

0 means failure, other values are used as the handle of functions such as NETDEV_StopDownloadFile. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_StopDownloadFile

Stop downloading recordings.

int NETDEV_StopDownloadFile(

long lpPlayHandle

);

Parameter

lpPlayHandle

[IN] Playback handle, return value of NETDEV_DownloadFile

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Non-Live View Capture

### NETDEV_CaptureNoPreview

int NETDEV_CaptureNoPreview(

long lpUserID,

int dwChannelID,

int dwStreamType,

String pszFileName,

int dwCaptureMode

);

Parameter

lpUserID

[IN] User login handle

dwChannelID

[IN] Channel ID

dwStreamType

[IN] Stream type

pszFileName

[IN] Path to the saved image (including filename)

dwCaptureMode

[IN] Image format

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

dwStreamType

| Value | Description |
| --- | --- |
| 0 | Main stream |
| 1 | Sub stream |
| 2 | Third stream |

## Display Service

### Initialize

Initialize display service.

int initialize(

);

Parameter

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### setRendererViewport

Set video output size.

int setRendererViewport(

int    w,

int    h

);

Parameter

W

[IN] Video output width

H

[IN] Video output height

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### initializeRenderer

Initialize renderer.

int initializeRenderer(

int dwWinIndex

);

Parameter

dwWinIndex

[IN] Window index

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### rendererRender

Render video data.

int rendererRender(

int dwWinIndex

);

Parameter

dwWinIndex

[IN] Window index

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### Scale

Adjust digital zoom.

int Scale(float scaleRatio,

float x,

float y,

int dwWinIndex

);

Parameter

scaleRatio

[IN] Scale

X

[IN] X coordinate

y

[IN] Y coordinate

dwWinIndex

[IN] Window index

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Time Settings

### NETDEV_GetSystemTimeCfg

Get device system time configuration.

int NETDEV_GetSystemTimeCfg(

long lpUserID,

NETDEV_TIME_S stDeviceTime

);

Parameter

lpUserID

[IN] User login ID

stDeviceTime

[IN] Time configuration info

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_SetSystemTimeCfg

Set device system time.

int NETDEV_SetSystemTimeCfg (

long lpUserID,

NETDEV_TIME_S stDeviceTime

);

Parameter

lpUserID

[IN] User login ID

stDeviceTime

[IN] Time configuration info

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## PTZ Control

### NETDEV_PTZControl

Control PTZ (start live view first).

Int NETDEV_PTZControl(

long lpPlayID,

int dwPTZCommand,

int dwSpeed

);

Parameter

lpPlayID

[IN] Live view ID

dwPTZCommand

[IN] PTZ control commands

| Value | Description |
| --- | --- |
| 0x0101 | Stop closing iris |
| 0x0102 | Close iris |
| 0x0103 | Stop opening iris |
| 0x0104 | Open iris |
| 0x0201 | Stop focusing near |
| 0x0202 | Focus near |
| 0x0203 | Stop focusing far |
| 0x0204 | Focus far |
| 0x0301 | Stop zooming in |
| 0x0302 | Zoom in |
| 0x0303 | Stop zooming out |
| 0x0304 | Zoom out |
| 0x0402 | Tilt up |
| 0x0404 | Tilt down |
| 0x0502 | Pan right |
| 0x0504 | Pan left |
| 0x0702 | Pan left and tilt up |
| 0x0704 | Pan left and tilt down |
| 0x0802 | Pan right and tilt up |
| 0x0804 | Pan right and tilt down |
| 0x0901 | Full stop |
| 0x0907 | Stop focus and iris |
| 0x0908 | Stop moving |
| 0x0909 | Stop zoom |
| 0x1001 | Start recorded patrol (by a recorded patrol route) |
| 0x1002 | Stop recorded patrol (by a recorded patrol route) |
| 0x1003 | Start recording a patrol route |
| 0x1004 | Stop recording a patrol route |
| 0x1005 | Add patrol route |
| 0x1006 | Delete patrol route |
| 0x1101 | Drag to zoom in |
| 0x1102 | Drag to zoom out |
| 0x1103 | 3D positioning |
| 0x0A01 | Enable wiper |
| 0x0A02 | Disable wiper |
| 0x0B01 | Enable light |
| 0x0B02 | Disable light |
| 0x0C01 | Enable heater |
| 0x0C02 | Disable heater |
| 0x0E01 | Enable snow removal |
| 0x0E02 | Disable snow removal |
| 0x0D01 | Enable IR |
| 0x0D02 | Disable IR |
| 0x1801 | Self-test: The PTZ performs a self-test when dwSpeed=1; when dwSpeed=0, PTZ physical parameters are restored to defaults and a restart will occur. |

dwSpeed

[IN] PTZ control speed, which is set by user according to the decoder's speed control value; range: [1, 9].

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_PTZControl_Other

Control PTZ (start live view first).

Int NETDEV_PTZControl_Other (

long lpUserID,

int dwChannelID,

int dwPTZCommand,

int dwSpeed

);

Parameter

lpUserID

[IN] User login handle

dwChannelID

[IN] Channel ID

dwPTZCommand

[IN] PTZ control commands

| Value | Description |
| --- | --- |
| 0x0101 | Stop closing iris |
| 0x0102 | Close iris |
| 0x0103 | Stop opening iris |
| 0x0104 | Open iris |
| 0x0201 | Stop focusing near |
| 0x0202 | Focus near |
| 0x0203 | Stop focusing far |
| 0x0204 | Focus far |
| 0x0301 | Stop zooming in |
| 0x0302 | Zoom in |
| 0x0303 | Stop zooming out |
| 0x0304 | Zoom out |
| 0x0402 | Tilt up |
| 0x0404 | Tilt down |
| 0x0502 | Pan right |
| 0x0504 | Pan left |
| 0x0702 | Pan left and tilt up |
| 0x0704 | Pan left and tilt down |
| 0x0802 | Pan right and tilt up |
| 0x0804 | Pan right and tilt down |
| 0x0901 | Full stop |
| 0x0907 | Stop focus and iris |
| 0x0908 | Stop moving |
| 0x0909 | Stop zoom |
| 0x1001 | Start recorded patrol (by a recorded patrol route) |
| 0x1002 | Stop recorded patrol (by a recorded patrol route) |
| 0x1003 | Start recording a patrol route |
| 0x1004 | Stop recording a patrol route |
| 0x1005 | Add patrol route |
| 0x1006 | Delete patrol route |
| 0x1101 | Drag to zoom in |
| 0x1102 | Drag to zoom out |
| 0x1103 | 3D positioning |
| 0x0A01 | Enable wiper |
| 0x0A02 | Disable wiper |
| 0x0B01 | Enable light |
| 0x0B02 | Disable light |
| 0x0C01 | Enable heater |
| 0x0C02 | Disable heater |
| 0x0E01 | Enable snow removal |
| 0x0E02 | Disable snow removal |
| 0x0D01 | Enable IR |
| 0x0D02 | Disable IR |
| 0x1801 | Self-test: The PTZ performs a self-test when dwSpeed=1; when dwSpeed=0, PTZ physical parameters are restored to defaults and a restart will occur. |

dwSpeed

[IN] PTZ control speed, which is set by user according to the decoder's speed control value; range: [1, 9].

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_GetPTZPresetList

Get preset list.

int NETDEV_GetPTZPresetList(

long lpUserID,

int dwChannelID,

int[] dwPresetIDList

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

dwPresetIDList

[OUT] Preset ID list

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_GetPTZPresetListEx

Get preset list.

NETDEV_PTZ_PRESET_S[] NETDEV_GetPTZPresetListEx (

long lpUserID,

int dwChannelID

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

Return Value

NULL means failure, other values mean preset ID list.

Remarks

### NETDEV_PTZPreset

Control PTZ preset (start live view first).

int NETDEV_PTZPreset(

long 	lpPlayID,

int  	dwPTZPresetCmd,

String  szPresetName,

int 	dwPresetID

);

Parameter

lpPlayID

[IN] Live view ID

dwPTZPresetCmd

[IN] Preset control commands

| Value | Description |
| --- | --- |
| 0 | Set preset |
| 1 | Clear preset |
| 2 | Go to preset |

szPresetName

[IN] Preset name

dwPresetID

[IN] Preset ID

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_PTZPreset_Other

Control PTZ preset.

Int NETDEV_PTZPreset_Other(

int 	lpUserID,

int 	dwChannelID,

int  	dwPTZPresetCmd,

String 	szPresetName,

int 	dwPresetID

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

dwPTZPresetCmd

[IN] Preset control commands

| Value | Description |
| --- | --- |
| 0 | Set preset |
| 1 | Clear preset |
| 2 | Go to preset |

szPresetName

[IN] Preset name

dwPresetID

[IN] Preset ID

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_PTZAbsoluteMove

Absolute PTZ coordinates movement.

int NETDEV_PTZAbsoluteMove(

long lpUserID,

int dwChannelID,

NETDEV_PTZ_ABSOLUTE_MOVE_S stPTZAbsoluteMove

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

stPTZAbsoluteMove

[IN] Struct info of absolute PTZ coordinates movement

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_PTZGetStatus

Absolute PTZ coordinates movement.

int NETDEV_PTZGetStatus(

long lpUserID,

int dwChannelID,

NETDEV_PTZ_STATUS_S stPTZStatus

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

stPTZStatus

[IN] PTZ status

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Two-way Audio

### NETDEV_StartInputVoiceSrv

Start audio data input service.

long NETDEV_StartInputVoiceSrv(

long lpUserID,

int dwChannelID

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

Return Value

NULL means failure, other values are used as parameters of NETDEV_InputVoiceData and NETDEV_StopInputVoiceSrv.

Remarks

### NETDEV_Android_StartInputVoiceSrv

Start audio data input service (including creating stream callback).

long NETDEV_Android_StartInputVoiceSrv(

long lpUserID,

int dwChannelID

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

Return Value

NULL means failure, other values are used as parameters of NETDEV_InputVoiceData and NETDEV_Android_StopInputVoiceSrv.

Remarks

### NETDEV_InputVoiceData

Input audio data.

int NETDEV_InputVoiceData(

long lpVoiceComHandle,

byte[] lpDataBuf,

int dwDataLen,

NETDEV_AUDIO_SAMPLE_PARAM_S stVoiceParam

);

Parameter

lpVoiceComHandle

[IN] Two-way audio handle

lpDataBuf

[IN] Audio data address

dwDataLen

[IN] Audio data length

stVoiceParam

[IN] Audio parameter

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_StopInputVoiceSrv

Stop audio data input service.

int NETDEV_StopInputVoiceSrv(

long lpVoiceComHandle

);

Parameter

lpVoiceComHandle

[IN] Two-way audio handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_Android_StopInputVoiceSrv

Stop audio data input service (including deleting stream callback).

int NETDEV_Android_StopInputVoiceSrv (

long lpVoiceComHandle

);

Parameter

lpVoiceComHandle

[IN] Two-way audio handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Organization Info

### NETDEV_FindOrgInfoList

Query organization list.

long NETDEV_FindOrgInfoList(

long iUserID,

NETDEV_ORG_FIND_COND_S stFindCond

);

Parameter

iUserID

[IN] User login ID

stFindCond

[IN] Query criteria

Return Value

Query handle. 0 means failure, other values are used as parameters of functions such as NETDEV_FindNextOrgInfo and NETDEV_FindCloseOrgInfo.

Remarks

### NETDEV_FindNextOrgInfo

Get organization info one by one.

long NETDEV_FindNextOrgInfo (

long iFindHandle,

NETDEV_ORG_INFO_S stOrgInfo

);

Parameter

iFindHandle

[IN] Query handle

stOrgInfo

[IN] Organization info

Return Value

TRUE means success, other values mean failure. Failure means query ended.

Remarks

### NETDEV_FindCloseOrgInfo

Close organization info query and release the resource.

long NETDEV_FindCloseOrgInfo (

long iFindHandle,

);

Parameter

iFindHandle

[IN] Query handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_FindOrgChnList

Query channel list by organization ID.

long NETDEV_FindOrgChnList (

long iUserID,

int dwOrgID,

int dwChnType

);

Parameter

iUserID

[IN] User login ID

dwOrgID

[IN] Organization ID

dwChnType

[IN] Channel type

Return Value

0 means failure, other values are used as parameters of functions such as NETDEV_FindNextOrgChn and NETDEV_FindCloseOrgChn.

Remarks

dwChnType

| Value | Description |
| --- | --- |
| 0 | Encoding channel |
| 1 | Alarm input channel |
| 2 | Alarm output channel |
| 3 | Decoding channel |
| 4 | Audio channel |
| 5 | NIC channel |
| 6 | Alarm device |
| 7 | Door |
| 8 | ADU local encoding channel |
| 9 | Emergency bell channel |

### NETDEV_FindNextOrgChn

Get channel info of an organization one by one.

int NETDEV_FindNextOrgChn(

long iUserID,

NETDEV_ORG_CHN_INFO_S stOrgChnInfo

);

Parameter

iUserID

[IN] Query handle

stOrgChnInfo

[IN] Organization channel info

Return Value

TRUE means success, other values mean failure. Failure means query ended.

Remarks

### NETDEV_FindCloseOrgChn

Close the query of channel info of an organization and release the resource.

int NETDEV_FindCloseOrgChn(

long iFindHandle

);

Parameter

iFindHandle

[IN] Query handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Device Info

### NETDEV_GetDeviceInfo

Get device info.

int NETDEV_GetDeviceInfo(

long lpUserID,

int dwChannelID,

NETDEV_DEVICE_BASICINFO_S stDevinfo

);

Parameter

lpUserID

[IN] User login ID

dwChannelID

[IN] Channel ID

stDevinfo

[OUT] Device info

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_FindDevList

Query device list by device type.

long NETDEV_FindDevList(

long iUserID,

int dwDevType

);

Parameter

iUserID

[IN] User login ID

dwDevType

[IN] Device type

Return Value

Query handle. 0 means failure, other values are used as parameters of functions such as NETDEV_ FindNextDevInfo and NETDEV_ FindCloseDevInfo.

Remarks

dwDevType

| Value | Description |
| --- | --- |
| 0 | Encoding device |
| 1 | Decoding device |
| 2 | VMS |
| 3 | Agent |
| 4 | Cloud encoding device |
| 5 | License plate or face recognition device |
| 6 | Access control device |
| 7 | Alarm control device |

### NETDEV_FindNextDevInfo

Get device info one by one.

int NETDEV_FindNextDevInfo(

long iUserID,

NETDEV_DEV_BASIC_INFO_S stDevBasicInfo

);

Parameter

iUserID

[IN] Query handle

stDevBasicInfo

[IN] Basic device info

Return Value

TRUE means success, other values mean failure. Failure means query ended.

Remarks

### NETDEV_FindCloseDevInfo

Query device list by device type.

int NETDEV_FindCloseDevInfo(

long iFindHandle

);

Parameter

iFindHandle

[IN] Query handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_FindDevChnList

Query channel list by device ID or channel type.

long NETDEV_FindDevChnList (

long iUserID,

int dwDevID,

int dwChnType

);

Parameter

iUserID

[IN] User login ID

dwDevID

[IN] Device ID

dwChnType

[IN] Channel type

Return Value

Query handle. 0 means failure, other values are used as parameters of functions such as NETDEV_FindNextDevChn and NETDEV_FindCloseDevChn.

Remarks

dwChnType

| Value | Description |
| --- | --- |
| 0 | Encoding channel |
| 1 | Alarm input channel |
| 2 | Alarm output channel |
| 3 | Decoding channel |
| 4 | Audio channel |
| 5 | NIC channel |
| 6 | Alarm device |
| 7 | Door |
| 8 | ADU local encoding channel |
| 9 | Emergency bell channel |

### NETDEV_FindNextDevChn

Get channel info of a device one by one.

int NETDEV_FindNextDevChn(

long iFindHandle,

NETDEV_DEV_CHN_ENCODE_INFO_S stDevChnEncodeInfo

);

Parameter

iFindHandle

[IN] Query handle

stDevChnEncodeInfo

[IN] Encoding channel info of device

Return Value

TRUE means success, other values mean failure. Failure means query ended.

Remarks

### NETDEV_FindCloseDevChn

Close the query of channel info of a device and release the resource.

int NETDEV_FindCloseDevChn(

long iFindHandle

);

Parameter

iFindHandle

[IN] Query handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Configuration Info

### NETDEV_GetDevConfig

Get device configuration info.

int NETDEV_GetDevConfig(

long iUserID,

int iChannelID,

int iCommand,

Object oOutBuffer,

int iOutBufferSize,

int iBytesReturned

);

Parameter

iUserID

[IN] User login handle

iChannelID

[IN] Channel ID

iCommand

[IN] Device configuration command

oOutBuffer

[INOUT] Pointer to received data

iOutBufferSize

[OUT] Buffer size of received data (unit: byte), cannot be 0

iBytesReturned

[OUT] Pointer to data size, cannot be NULL

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

iCommand

| Value | Description |
| --- | --- |
| 100 | Get device info See NETDEV_DEVICE_BASICINFO_S |

### NETDEV_SetDevConfig

Get device configuration info.

int NETDEV_SetDevConfig(

long iUserID,

int iChannelID,

int iCommand,

Object oInBuffer,

int iInBufferSizes

);

Parameter

iUserID

[IN] User login handle

iChannelID

[IN] Channel ID

iCommand

[IN] Device configuration command

oInBuffer

[INOUT] Pointer to input data

iInBufferSize

[IN] Buffer size of input data (unit: byte), cannot be 0

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Register Callback

### NETDEV_SetAlarmCallBackV2

Register callback function for receiving alarm messages.

int  NETDEV_SetAlarmCallBackV2(

long iUserID,

String strAlarmMessCallBack,

int iUserData

);

Parameter

iUserID

[IN] User login handle

strAlarmMessCallBack

[IN] Alarm callback function

iUserData

[IN] User data

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_Android_SetAlarmCallBack

Register callback function for receiving alarm messages.

int  NETDEV_SetAlarmCallBackV2(

long iUserID,

AlarmCallBack_PF strAlarmMessCallBack,

int iUserData

);

Parameter

iUserID

[IN] User login handle

strAlarmMessCallBack

[IN] Alarm callback function

iUserData

[IN] User data

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_SetAlarmCallBack_V30

Register callback function for receiving alarm messages.

int  NETDEV_SetAlarmCallBackV2(

long iUserID,

String strAlarmMessCallBackV30,

int iUserData

);

Parameter

iUserID

[IN] User login handle

strAlarmMessCallBackV30

[IN] Alarm callback function

iUserData

[IN] User data

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_Android_SetAlarmCallBack_V30

Register callback function for receiving alarm messages.

int  NETDEV_Android_SetAlarmCallBack_V30 (

long iUserID,

AlarmMessCallBackV30 strAlarmMessCallBackV30,

int iUserData

);

Parameter

iUserID

[IN] User login handle

strAlarmMessCallBackV30

[IN] Alarm callback function

iUserData

[IN] User data

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_SetExceptionCallBack

Register callback function for receiving exception and reconnection messages.

int  NETDEV_SetExceptionCallBack (

String strExceptionCallBack,

long iUserData

);

Parameter

strExceptionCallBack

[IN] Callback function for receiving exception messages

iUserData

[IN] User data

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_Android_SetExceptionCallBack

Register callback function for receiving exception and reconnection messages.

int  NETDEV_Android_SetExceptionCallBack (

ExceptionCallBack_PF strExceptionCallBack,

long iUserData

);

Parameter

strExceptionCallBack

[IN] Callback function for receiving exception messages

iUserData

[IN] User data

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Alarm Subscription

### NETDEV_SubscibeLapiAlarm

Subscribe to LAPI alarms.

int NETDEV_SubscibeLapiAlarm(

long lpUserID,

NETDEV_LAPI_SUB_INFO_S stSubInfo,

NETDEV_SUBSCRIBE_SUCC_INFO_S stSubSuccInfo

);

Parameter

lpUserID

[IN] User login handle

stSubInfo

[IN] Alarm subscription request

stSubSuccInfo

[IN] Subscription success info

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_UnSubLapiAlarm

Unsubscribe from LAPI alarms.

int NETDEV_UnSubLapiAlarm(

long lpUserID,

int udwID

);

Parameter

lpUserID

[IN] User login handle

udwID

[IN] Alarm subscription ID

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Query People Flow

### NETDEV_StartMultiTrafficStatistic

Start querying people flow statistics for multiple channels.

int NETDEV_StartMultiTrafficStatistic(

long lpUserID,

NETDEV_MULTI_TRAFFIC_STATISTICS_COND_S pstStatisticCond,

MutableInteger pudwSearchID

);

Parameter

lpUserID

[IN] User login handle

pstStatisticCond

[IN] People flow statistics struct

pudwSearchID

[OUT] People flow query ID

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_GetTrafficStatisticProgress

Get people flow counting progress.

int NETDEV_GetTrafficStatisticProgress(

long lpUserID,

int udwSearchID,

MutableInteger pudwProgress

);

Parameter

lpUserID

[IN] User login handle

udwSearchID

[IN] People flow query ID

pudwProgress

[OUT] People flow counting progress

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_StopTrafficStatistic

Stop people flow query.

int NETDEV_StopTrafficStatistic(

long lpUserID,

int udwSearchID

);

Parameter

lpUserID

[IN] User login handle

udwSearchID

[IN] People flow query ID

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_FindTrafficStatisticInfoList

Get people flow statistics list.

long NETDEV_FindTrafficStatisticInfoList(

long lpUserID,

int udwSearchID

);

Parameter

lpUserID

[IN] User login handle

udwSearchID

[IN] People flow query ID

Return Value

0 means failure, other values are used as parameters of functions such as NETDEV_FindNextTrafficStatisticInfo and NETDEV_FindCloseTrafficStatisticInfo.

Remarks

### NETDEV_FindNextTrafficStatisticInfo

Get people flow info one by one.

int NETDEV_FindNextTrafficStatisticInfo(

long lpFindHandle,

NETDEV_TRAFFIC_STATISTICS_INFO_S pstStatisticInfo

);

Parameter

lpFindHandle

[IN] Query handle

pstStatisticInfo

[IN] People flow query info

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_FindCloseTrafficStatisticInfo

Close people flow query and release the resource.

int NETDEV_FindCloseTrafficStatisticInfo(

long lpFindHandle

);

Parameter

lpFindHandle

[IN] Query handle

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_SetPassengerFlowStatisticCallBack

Register callback function for receiving people flow counting messages.

int NETDEV_SetPassengerFlowStatisticCallBack(

long lpUserID,

String strPassengerFlowStatisticCB,

long iUserData

);

Parameter

lpUserID

[IN] User login handle

pstPassengerFlowData

[IN] People flow data

lpUserData

[IN] User data

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Device Discovery

### NETDEV_Discovery

Register callback function for discovering devices first and then call this function to discover devices.

boolean NETDEV_Discovery(

String szBeginIP,

String szEndIP

);

Parameter

szBeginIP

[IN] Start IP

szEndIP

[IN] End IP

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_SetDiscoveryCallBack

public interface NETDEV_DISCOVERY_CALLBACK_PF {

public void invoke(NETDEV_DISCOVERY_DEVINFO_S stDevInfo);

}

Register callback function for device discovery.

boolean NETDEV_SetDiscoveryCallBack(

NETDEV_DISCOVERY_CALLBACK_PF cbDiscoveryCallBack

);

Parameter

cbDiscoveryCallBack

[IN] Callback function

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

### NETDEV_ModifyDeviceAddr

Modify device IP.

boolean NETDEV_ModifyDeviceAddr(

NETDEV_DEV_ADDR_INFO_S  stDevAddrInfo

);

Parameter

stDevAddrInfo

[IN] The new device IP to set

Return Value

TRUE means success, other values mean failure. Call NETDEV_GetLastError() to get the error code. See Error Code List for details.

Remarks

## Get Error Code

### NETDEV_GetLastError

Get error code.

Int NETDEV_GetLastError(

);

Parameter

Return Value

The corresponding error code.

Remarks

### Error Code List

| Error Code | Value | Indication |
| --- | --- | --- |
| NETDEV_E_FAILED | -1 | Failure |
| NETDEV_E_SUCCEED | 0 | Success |
| NETDEV_E_SVC_FAILED | 1 | Server failure |
| NETDEV_E_NO_USER | 5 | User does not exist |
| NETDEV_E_NO_RESULT | 11 | No result found |
| NETDEV_E_NOENOUGH_BUF | 12 | Buffer for receiving device data is too small |
| NETDEV_E_SDK_SOCKET_LSN_FAIL | 13 | Failed to create socket listen |
| NETDEV_E_INIT_MUTEX_FAIL | 14 | Failed to initialize lock |
| NETDEV_E_INIT_SEMA_FAIL | 15 | Failed to initialize semaphore |
| NETDEV_E_ALLOC_RESOURCE_ERROR | 16 | SDK resource allocation error |
| NETDEV_E_HAVEDATA | 17 | Failed to send data |
| NETDEV_E_NEEDMOREDATA | 18 | Insufficient memory allocated |
| NETDEV_E_TRANSFILE_FAIL | 19 | Failed to transfer file |
| NETDEV_E_DEVICE_TYPE_ERR | 20 | Unsupported device type |
| NETDEV_E_NONCE_TIMEOUT | 21 | nonce expired |
| NETDEV_E_INNER_ERR | 22 | System internal error |
| NETDEV_E_BINDNOTIFY_FAIL | 24 | Failed to bind alarm |
| NETDEV_E_SYSCALL_FALIED | 100 | Failed to call system function |
| NETDEV_E_NULL_POINT | 101 | Null pointer |
| NETDEV_E_INVALID_PARAM | 102 | Invalid parameter |
| NETDEV_E_INVALID_MODULEID | 103 | Invalid module ID |
| NETDEV_E_INVALID_HANDLE | 104 | Invalid handle |
| NETDEV_E_NO_MEMORY | 105 | Failed to allocate memory |
| NETDEV_E_FILE_NO_EXIST | 106 | File does not exist |
| NETDEV_E_NO_DEV | 107 | Device does not exist |
| NETDEV_E_NO_FIT_LOG | 108 | No matching log |
| NETDEV_E_BUSY | 109 | Busy status |
| NETDEV_E_TIMER_REG_FAILED | 110 | Failed to register timer |
| NETDEV_E_COMMON_FAILED | 111 | Common error |
| NETDEV_E_CMD_NOT_SUPPORT | 112 | Command not supported |
| NETDEV_E_NOT_SUPPORT | 113 | Device does not support the function |
| NETDEV_E_TIMEOUT | 114 | Timeout |
| NETDEV_E_MSG_ERR | 115 | Message does not match |
| NETDEV_E_MODULE_INEXIST | 116 | Module does not exist |
| NETDEV_E_SOCKET_RECV_ERR | 117 | Failed to receive message |
| NETDEV_E_DECODE_IE_FAILED | 118 | Failed to get message IE |
| NETDEV_E_ENCODE_IE_FAILED | 119 | Failed to add message IE |
| NETDEV_E_SDK_NOINTE_ERROR | 120 | SDK is not initialized |
| NETDEV_E_ALREDY_INIT_ERROR | 121 | SDK is initialized |
| NETDEV_E_DEVICE_FACTURER_ERR | 122 | Unsupported device manufacturer |
| NETDEV_E_NAME_EXIST | 123 | Name already exists |
| NETDEV_E_GET_CFG_FAILED | 124 | Error occurs while getting configurations |
| NETDEV_E_SET_CFG_FAILED | 125 | Error occurs while setting configurations |
| NETDEV_E_CHANNEL_OVER_SPEC | 126 | Number of channels exceeds the upper limit |
| NETDEV_E_CALL_DRV_COMMON | 127 | Common driver calling failure |
| NETDEV_E_TOTAL_QUOTA_FULL | 128 | Insufficient space quota to allocate |
| NETDEV_E_CALL_DB_COMMON | 129 | Common database calling failure |
| NETDEV_E_NEED_MORE_MEMORY | 130 | Insufficient memory |
| NETDEV_E_T2U_CONNECT_FAILED | 131 | T2U connection failed |
| NETDEV_E_FUNC_IS_INITIALIZING | 132 | Function is initializing |
| NETDEV_E_CONNECT_ERROR | 200 | Failed to establish a connection |
| NETDEV_E_SEND_MSG_ERROR | 201 | Failed to send message |
| NETDEV_E_DECODE_RSP_ERROR | 202 | Failed to parse response message |
| NETDEV_E_NONSUPPORT | 203 | Unsupported function |
| NETDEV_E_JSON_ERROR | 204 | Common Json error |
| NETDEV_E_NORESULT | 205 | No search results |
| NETDEV_E_SOCKET_RECV_ERROR | 206 | Socket failed to receive message |
| NETDEV_E_CREATE_THREAD_FAIL | 207 | Failed to create thread |
| NETDEV_E_RESCODE_NO_EXIST | 208 | Resource code does not exist |
| NETDEV_E_MSG_DATA_INVALID | 209 | Message content error |
| NETDEV_E_JSON_NO_IMAGE | 210 | No image |
| NETDEV_E_PUBLICKEYFAIL | 60068 | Failed to pair public key |
| NETDEV_E_PLAYER_FAIL | 1001 | Execution failure |
| NETDEV_E_PLAYER_INVALID_PARAM | 1002 | Invalid input parameter |
| NETDEV_E_PLAYER_NO_MEMORY | 1003 | Insufficient system memory |
| NETDEV_E_PLAYER_SOCKET_FAIL | 1004 | Failed to create SOCKET |
| NETDEV_E_PLAYER_RECV_FAIL | 1005 | Failed to receive |
| NETDEV_E_PLAYER_RECV_ZERO | 1006 | Nothing received |
| NETDEV_E_PLAYER_NOT_SUPPORT | 1007 | Unsupported function |
| NETDEV_E_PLAYER_CREATETHREAD_FAILED | 1008 | Failed to create thread |
| NETDEV_E_PLAYER_OPENDL_FAILED | 1009 | Failed to load dynamic library |
| NETDEV_E_PLAYER_SYMDL_FAILED | 1010 | Failed to get dynamic method |
| NETDEV_E_PLAYER_SEND_FAILED | 1011 | Failed to send |
| NETDEV_E_PLAYER_EACCES | 1012 | No permission to create file |
| NETDEV_E_PLAYER_FILE_NOT_FIND | 1013 | No read file found |
| NETDEV_E_PLAYER_LOG_CLOSE | 1014 | Log off |
| NETDEV_E_PLAYER_MEDIA_EXCEPTION | 1017 | Internal processing exception |
| NETDEV_E_FAIL_TO_INIT_EZPLAYER | 1257 | Failed to initialize player |
| NETDEV_E_FAIL_TO_ALLOC_PORT_RES | 1258 | Failed to allocate playback channel resources |
| NETDEV_E_FAIL_TO_GET_PORT_RES | 1259 | Failed to get playback channel resources |
| NETDEV_E_BUFFER_QUEUE_FULL | 1260 | Buffer queue is full |
| NETDEV_E_BUFFER_QUEUE_EMPTY | 1261 | Buffer queue is empty |
| NETDEV_E_OPEN_FILE_FAILED | 1262 | Failed to open file |
| NETDEV_E_FILE_READ_END | 1263 | File read successfully |
| NETDEV_E_FILE_DISKSPACE_FULL | 1264 | Full disk space |
| NETDEV_E_FILE_READ_FAIL | 1265 | Failed to read |
| NETDEV_E_MCM_MIC_NOT_EXIST | 1266 | MIC does not exist |
| NETDEV_E_TS_PACKET_IN_THE_ROUGH | 1267 | TS packing is not finished |
| NETDEV_E_FILE_RECORD_FINISH | 1268 | Recording saved successfully |
| NETDEV_E_VIDEO_RESOLUTION_CHANGE | 1269 | Resolution changed |
| NETDEV_E_VIDEO_RECORD_PART | 1270 | Recording is segmented |
| NETDEV_E_FAIL_TO_OPEN_STREAM | 1513 | Failed to start playing stream |
| NETDEV_E_FAIL_TO_CLOSE_STREAM | 1514 | Failed to stop playing stream |
| NETDEV_E_FAIL_TO_RECV_DATA | 1515 | Failed to receive data due to network failure |
| NETDEV_E_FAIL_TO_PROCESS_MEDIA_DATA | 1516 | Failed to process media data |
| NETDEV_E_NOT_START_PLAY | 1517 | Playback channel does not start playback operation |
| NETDEV_E_FAIL_TO_INPUT_DATA | 1518 | Failed to input stream data |
| NETDEV_E_INPUTDATA_BUFFER_FULL | 1519 | Input data buffer is full |
| NETDEV_E_FAIL_TO_SET_PROCESS_DATA_CB | 1520 | Failed to set stream data callback function |
| NETDEV_E_VOICE_RUNNING | 1521 | Error occurred during audio service |
| NETDEV_E_FAIL_TO_OPEN_VOICE_SVC | 1522 | Failed to start audio service |
| NETDEV_E_FAIL_TO_CLOSE_VOICE_SVC | 1523 | Failed to stop audio service |
| NETDEV_E_UNKNOWN_STREAM_TYPE | 1524 | Unknown stream type |
| NETDEV_E_PACKET_LOSE | 1525 | Packet loss |
| NETDEV_E_NEED_MORE_PACKET | 1526 | More packets required to complete packing |
| NETDEV_E_FAIL_TO_CREATE_DECODE | 1527 | Failed to create decoder |
| NETDEV_E_FAIL_TO_DECODE | 1528 | Decoding failed |
| NETDEV_E_RECV_DATA_NOTENOUGH | 1529 | Insufficient data received |
| NETDEV_E_RENDER_RES_FULL | 1530 | Display resource is full |
| NETDEV_E_RENDER_RES_NOT_EXIST | 1531 | Display resource does not exist |
| NETDEV_E_CREATE_DEV_FAILED | 1532 | Failed to create resource |
| NETDEV_E_AUDIO_RES_NOT_EXIST | 1533 | Audio resource does not exist |
| NETDEV_E_IHW265D_NEED_MORE_BITS | 1534 | Decoder requires more data |
| NETDEV_E_FAIL_TO_CREATE_ENCODE | 1535 | Failed to create encoder |
| NETDEV_E_CAPTURE_RES_EXIST | 1536 | Capture resource does not exist |
| NETDEV_E_RECORD_STARTED | 1537 | Recording started |
| NETDEV_E_NEED_WAIT_DECODEC | 1538 | Decoding... Please wait |
| NETDEV_E_MORE_DATA_NEED_PACKET | 1539 | More data to be packed |
| NETDEV_E_AAC_LC_DECODE_FAIL | 1540 | AAC_LC decoding failed |
| NETDEV_E_RENDER_SURFACELOST | 1541 | Render surface lost |
| NETDEV_E_PLAYBACK_FINISH | 1542 | Playback finished |
| NETDEV_E_FILE_ENCRYPED | 1543 | File is encrypted |
| NETDEV_E_SCRAMBLING_INFO_FAILED | 1544 | Scrambling info failed |
| NETDEV_E_LIVE_EXISTED | 2000 | Live view has been established |
| NETDEV_E_LIVE_INPUT_NOT_READY | 2001 | Media stream is not ready |
| NETDEV_E_LIVE_OUTPUT_BUSY | 2002 | Live view display resource is busy |
| NETDEV_E_LIVE_CB_NOTEXIST | 2003 | Live view control module does not exist |
| NETDEV_E_LIVE_STREAM_FULL | 2004 | Live view stream resource is full |
| NETDEV_E_AUDIO_EXISTED | 2100 | Two-way audio already exists |
| NETDEV_E_AUDIO_NO_EXISTED | 2101 | Two-way audio does not exist |
| NETDEV_E_AUDIO_RESCODE_INVALID | 2102 | Invalid two-way audio resource code |
| NETDEV_E_AUDIO_RES_USED_BY_TALK | 2103 | Audio resource is being used by two-way audio |
| NETDEV_E_AUDIO_FAILED | 2104 | Two-way audio failed |
| NETDEV_E_AUDIO_AUDIOBCAST_FULL | 2205 | Audio service is full |
| NETDEV_E_CAPTURE_NO_SUPPORT_FORMAT | 2200 | Unsupported snapshot format |
| NETDEV_E_CAPTURE_NO_ENOUGH_CAPACITY | 2201 | Insufficient disk space |
| NETDEV_E_CAPTURE_NO_DECODED_PICTURE | 2202 | No decoded image available for snapshot |
| NETDEV_E_CAPTURE_SINGLE_FAILED | 2203 | Single snapshot failed |
| NETDEV_E_OVER_ABILITY | 2301 | Stream exceeds capability set |
| NETDEV_E_CLOUD_DOWNLOAD_FINISH | 2793 | Download finished |
| NETDEV_E_CLOUD_PARSE_DOMAIN_FAIL | 2794 | Failed to parse domain name |
| NETDEV_E_CLOUD_CONNECT_FAIL | 2795 | Connection failed |
| NETDEV_E_CLOUD_CONNECT_TIMEOUT | 2796 | Connection timed out |
| NETDEV_E_CLOUD_DOWNLOAD_TIMEOUT | 2797 | Download timed out |
| NETDEV_E_CLOUD_DOWNLOAD_FAIL | 2798 | Download failed |
| NETDEV_E_CLOUD_NETWORK_POOR | 2799 | Poor network |
| NETDEV_E_CLOUD_PLAY_FINISH | 2800 | Playback finished |
| NETDEV_E_CLOUD_DISK_FULL | 2801 | Full disk space |
| NETDEV_E_CLOUD_AUTH_FAIL | 2802 | Authentication failed |
| NETDEV_E_CLOUD_CURRENT_TIME | 2803 | Current playing time, used for reporting only |
| NETDEV_E_CLOUD_PRIOR_DISK_FULL | 2804 | Disk is full |
| NETDEV_E_CLOUD_NODE_NOT_EXIST | 2805 | Time node does not exist |
| NETDEV_E_CLOUD_NO_CACHE_PATH | 2806 | No cache path |
| NETDEV_E_CLOUD_MSG_SEND_FAIL | 2807 | Failed to send message |
| NETDEV_E_CLOUD_TASK_CANCELLED | 2808 | Task is cancelled |
| NETDEV_E_CLOUD_TASK_STREAM_CONTINUE | 2809 | Stream continues to play |
| NETDEV_E_MEDIA_INPUT_NOT_READY | 10000 | Media stream is not ready |
| NETDEV_E_CCB_STATR_INVALID | 10001 | Control block status is not available |
| NETDEV_E_MEDIA_OUTPUT_BUSY | 10002 | Live view display resource is busy |
| NETDEV_E_MEDIA_START_LOCAL_LIVE_ERR | 10003 | Live view stream is not ready |
| NETDEV_E_MEDIA_START_LOCAL_REPLAY_ERR | 10004 | Playback stream is not ready |
| NETDEV_E_MEDIA_BW_RECV_NOT_ENOUGH | 10007 | Insufficient network bandwidth for receiving data |
| NETDEV_E_MEDIA_BW_SEND_NOT_ENOUGH | 10008 | Insufficient network bandwidth for sending data |
| NETDEV_E_MEDIA_AUDIO_BROADCAST_TO_LIMIT | 10009 | Audio broadcast service has reached the upper limit |
| NETDEV_E_MEDIA_AUDIO_CHL_BING_USED | 10010 | Audio channel is being used |
| NETDEV_E_MEDIA_NOT_SUPPORT_ENCODETYPE | 10012 | Unsupported stream format |
| NETDEV_E_MEDIA_MAX | 10399 | Max error code for media |
| NETDEV_E_SET_PRESET_FAILED | 10400 | Failed to set preset |
| NETDEV_E_QUERY_PRESET_FAILED | 10401 | Failed to query preset |
| NETDEV_E_QUERY_TRACK_FAILED | 10402 | Failed to query route |
| NETDEV_E_START_RECORD_TRACK_FAILED | 10403 | Failed to start recording route |
| NETDEV_E_STOP_RECORD_TRACK_FAILED | 10404 | Failed to stop recording route |
| NETDEV_E_QUERY_CRUISE_FAILED | 10405 | Failed to query patrol route |
| NETDEV_E_SET_CRUISE_FAILED | 10406 | Failed to set patrol route |
| NETDEV_E_PTZ_COMMAND_FAILED | 10407 | PTZ operation failed |
| NETDEV_E_PRESET_IN_CRUISE | 10408 | Preset is being used by patrol route and cannot be deleted |
| NETDEV_E_CRUISEPOINT_ER | 10409 | Discontinuous patrol points |
| NETDEV_E_TRACK_ISUSED | 10410 | Route is being used and cannot be deleted |
| NETDEV_E_SERIALMODE_MISMATCH | 10411 | Serial port mode does not match |
| NETDEV_E_TRACK_NOT_EXIST | 10412 | Route does not exist |
| NETDEV_E_MODE_CRUISE_FULL | 10413 | Route points has reached the upper limit |
| NETDEV_E_PTZ_ISUSED | 10414 | PTZ is in use |
| NETDEV_E_PRESET_IN_GUARD | 10415 | Preset is being used by auto guard |
| NETDEV_E_CRUISE_PATH_IN_GUARD | 10416 | Patrol route is being used by auto guard |
| NETDEV_E_PTZ_MAX | 10799 | Max error code of PTZ module |
| NETDEV_E_VOD_ABEND | 10800 | Playback ended abnormally |
| NETDEV_E_VOD_END | 10801 | Playback ended |
| NETDEV_E_VOD_CALL_DRV | 10802 | Failed to call driver interface |
| NETDEV_E_VOD_CB_NOT_FIND | 10803 | Control block is not found |
| NETDEV_E_VOD_OVER_ABILITY | 10804 | Exceeded playback capability |
| NETDEV_E_VOD_TAG_OVER_LIMIT | 10805 | Number of tags exceeds the upper limit |
| NETDEV_E_VOD_NO_RECORDING | 10806 | No recording |
| NETDEV_E_VOD_SMART_NOT_SUPPORT | 10807 | Device connected via GB/T28181 does not support smart playback |
| NETDEV_E_VOD_NO_REPLAYURL | 10808 | Cannot get playback URL |
| NETDEV_E_VOD_MAX | 10999 | Max error code of playback module |
| NETDEV_E_STOR_ABEND | 101000 | Storage ended abnormally |
| NETDEV_E_ARRAY_NAME_EXITS | 11001 | Array already exists |
| NETDEV_E_NOT_RAID_MODE | 11002 | Non-RAID mode |
| NETDEV_E_NO_STORE_PLAN | 11003 | No recording schedule has been configured for this channel |
| NETDEV_E_DISK_ZERO_OR_ABNORMAL | 11022 | No hard disk or disk is abnormal |
| NETDEV_E_SLOT_NOT_NORMAL | 11023 | Disk in slot 1 is in abnormal status |
| NETDEV_E_SLOT_RECONNECT | 11024 | Restart is required because disk in slot 1 is inserted again after face recognition is enabled |
| NETDEV_E_BATTERY_STATUS_FAULT | 11034 | Device battery is abnormal |
| NETDEV_E_STOR_MAX | 11199 | Max error code of storage module |
| NETDEV_E_USER_NOT_ONLINE | 101200 | User is offline |
| NETDEV_E_USER_WRONG_PASSWD | 11201 | Incorrect user password |
| NETDEV_E_USER_NO_SUCH_USER | 11202 | User does not exist |
| NETDEV_E_USER_NO_AUTH | 11203 | User has no permission |
| NETDEV_E_USER_MAX_NUM | 11204 | No more users can be added |
| NETDEV_E_USER_EXIST | 11205 | User already exists |
| NETDEV_E_USER_LOGIN_MAX_NUM | 11206 | Number of logged-in users has reached the upper limit |
| NETDEV_E_USER_LOCKED | 11207 | User is locked |
| NETDEV_E_USER_IS_ADMIN | 11208 | Operation is not allowed because the user is admin |
| NETDEV_E_USER_NOT_VALID_PERIOD | 11209 | Invalid period |
| NETDEV_E_USER_ROLE_BEYOND_RANGE | 11210 | Number of user roles exceeds the upper limit |
| NETDEV_E_USER_SELOG_IP_LOCKED | 11211 | User IP is locked for security |
| NETDEV_E_ROLE_EXIST | 11212 | Role already exists |
| NETDEV_E_USER_NOT_VALID_TIME | 11213 | Invalid time |
| NETDEV_E_USER_WEAK_PASSWD_LOGIN_FAIL | 11214 | Login failed, please change the password into a strong one locally and try again |
| NETDEV_E_USER_IS_TEMPPASSWD | 11217 | Login with temporary password |
| NETDEV_E_USER_TEMPPASSWD_LOGIN_FAIL | 11218 | Login on WAN with temporary password is denied |
| NETDEV_E_USER_WEAK_PASSWD_MODIFY | 11219 | Access denied for weak password |
| NETDEV_E_USER_IP_NO_AUTH | 11221 | Unauthorized login IP |
| NETDEV_E_USER_MAX | 11599 | Max error code of user module |
| NETDEV_E_DEV_NOT_ONLINE | 11600 | Device is offline |
| NETDEV_E_DEV_WRONG_PASSWD | 11601 | Incorrect device password |
| NETDEV_E_DEV_REPEAT_DEV | 11602 | Device already exists |
| NETDEV_E_DEV_NO_SUCH_DEV | 11603 | Device does not exist |
| NETDEV_E_DEV_NO_SUCH_CHL | 11604 | Channel does not exist |
| NETDEV_E_DEV_MAX_NUM | 11605 | No more devices can be added on VMS |
| NETDEV_E_DEV_LOGIN_MAX_NUM | 11606 | Maximum number of logged-in device |
| NETDEV_E_DEV_COMMON_FAILED | 11607 | Common error returned from device |
| NETDEV_E_DEV_NO_SUCH_DEV_OID | 11608 | No history device ID |
| NETDEV_E_DEV_DEL_FORBID | 11609 | Device cannot be deleted |
| NETDEV_E_CHL_OFFLINE | 11610 | Channel is offline |
| NETDEV_E_DEV_TYPE_UNMATCHED | 11611 | Device type does not match |
| NETDEV_E_DEV_REPEAT_DEV_NAME | 11612 | Duplicate device name |
| NETDEV_E_DEV_CFG_FAILED | 11613 | Failed to configure device |
| NETDEV_E_DEV_NO_RESULT | 11614 | No result |
| NETDEV_E_DEV_DEC_OFFLINE | 11617 | Decoding device is offline (11615 and 11616 are for external use by VMS) |
| NETDEV_E_DEV_VCA_ALARM_NO_PIC | 11661 | Failed to get VCA alarm image |
| NETDEV_E_COMMON_FAIL_STAT | 11801 | Failed to read file (directory) status |
| NETDEV_E_COMMON_FILE_NONEXIST | 11802 | File does not exist |
| NETDEV_E_COMMON_MKDIR_ERR | 11803 | Failed to create directory |
| NETDEV_E_SUBSCRIBE_FULL | 11804 | Subscription is full for current user (error code cannot return when subscription is full at device side) |
| NETDEV_E_UPGRADE_NOTADMIN | 11805 | Only admin can upgrade |
| NETDEV_E_UPGRADE_INVALID | 11806 | Upgrade (not started) |
| NETDEV_E_UPGRADE_INPROCESS | 11807 | Upgrade (processing) |
| NETDEV_E_UPGRADE_NOMEMORY | 11808 | Upgrade (insufficient memory) |
| NETDEV_E_UPGRADE_FILE_OPEN_ERR | 11809 | Upgrade (error occurred when opening mirror file) |
| NETDEV_E_UPGRADE_DEVICE_ERR | 11810 | Upgrade (flash error) |
| NETDEV_E_UPGRADE_BUSY | 11811 | Cannot load multiple upgrade processes simultaneously |
| NETDEV_E_UPGRADE_FAIL_TIMEOUT | 11812 | Upgrade timed out |
| NETDEV_E_INVALID_CONFIGFILE | 11813 | Invalid configuration file |
| NETDEV_E_STOR_RESOURCE_NOTINIT | 11814 | Storage resource is not allocated |
| NETDEV_E_DEV_MAX | 11999 | Max error code of device module |
| NETDEV_E_ORG_MAX_NUM | 12000 | Maximum number of organizations - No more organizations can be added |
| NETDEV_E_ORG_MAX_LEVEL | 12001 | Maximum number of organization levels |
| NETDEV_E_ORG_RESOURCE_EXIST | 12002 | Cannot delete organizations containing resources |
| NETDEV_E_ORG_EXIST | 12003 | Duplicate organization on the same branch |
| NETDEV_E_ORG_NO_SUCH_ORG | 12004 | Organization does not exist |
| NETDEV_E_ORG_IS_ROOT | 12005 | Operation is not allowed to the root organization |
| NETDEV_E_ORG_NO_SUCH_PORG | 12006 | Parent organization does not exist |
| NETDEV_E_ORG_SUB_EXIST | 12007 | Cannot delete organizations containing sub-organizations |
| NETDEV_E_ORG_MAX | 12399 | Maximum error code of organization module |
| NETDEV_E_TVWALL_WND_NUMLIMITED | 12400 | Number of windows exceeds upper limit |
| NETDEV_E_TVWALL_NUMLIMTED | 12401 | Number of video walls exceeds upper limit |
| NETDEV_E_TVWALL_LED_NUMLIMTED | 12402 | Number of virtual LEDs exceeds upper limit |
| NETDEV_E_TVWALL_LED_LINELIMTED | 12403 | Number of lines of a single virtual LED exceeds upper limit |
| NETDEV_E_TVWALL_SCENE_OVER_LIMIT | 12404 | Number of scenes of a single video wall exceeds upper limit |
| NETDEV_E_TVWALL_BASEPIC_NUMLIMITED | 12405 | Number of base pictures that the device can save exceeds upper limit |
| NETDEV_E_TVWALL_BASEPIC_SCENELIMITED | 12406 | Number of base pictures of a scene exceeds upper limit |
| NETDEV_E_TVWALL_DECODE_OUTRANGE | 12407 | Target position exceeds decoding capability |
| NETDEV_E_TVWALL_WND_OUTRANGE | 12408 | Window size exceeds upper limit |
| NETDEV_E_TVWALL_WND_RANGELIMITED | 12409 | Window/roaming position out of range |
| NETDEV_E_TVWALL_SCROLL_LED_NUMLIMITED | 12410 | Number of rolling virtual LEDs exceeds upper limit |
| NETDEV_E_TVWALL_WND_LOCKED | 12430 | Current window is locked |
| NETDEV_E_TVWALL_LED_NOTEXIST | 12431 | Virtual LED does not exist |
| NETDEV_E_TVWALL_SEQ_EXISTD | 12432 | Sequence already exists |
| NETDEV_E_TVWALL_WND_NOTEXIST | 12433 | Window does not exist |
| NETDEV_E_TVWALL_WND_EXIST | 12434 | Window already exists |
| NETDEV_E_TVWALL_SPILT_NOTEXIST | 12435 | Split window does not exist |
| NETDEV_E_TVWALL_WND_NOSPILT | 12436 | Window cannot be split |
| NETDEV_E_TVWALL_CHANNEL_UESED | 12437 | Decoding channel is in use by other video walls |
| NETDEV_E_TVWALL_WINDEXIST | 12438 | Window exists on video wall |
| NETDEV_E_TVWALL_NOTEXIST | 12439 | Video wall does not exist |
| NETDEV_E_TVWALL_SPLIT_INFOERR | 12440 | Incorrect split type |
| NETDEV_E_TVWALL_CHANNEL_NUMERR | 12411 | Incorrect number of channels |
| NETDEV_E_TVWALL_ENC_ERR | 12442 | Media stream is not ready |
| NETDEV_E_TVWALL_PERSCREEN_NUMLIMITED | 12443 | Number of windows per VO exceeds upper limit |
| NETDEV_E_TVWALL_MAX | 12799 | Max error code of video wall module |
| NETDEV_E_IPADDR_CONFLICT | 12800 | IP address conflict |
| NETDEV_E_UPNP_PORT_DISCARD | 12801 | Mapped port is discarded |
| NETDEV_E_UPNP_PORT_EMPLOY | 12802 | Port is already in use |
| NETDEV_E_SET_VMP_PORT_FAILED | 12803 | Failed to set VMP port |
| NETDEV_E_SET_MEDIA_PORT_FAILED | 12804 | Failed to set media port |
| NETDEV_E_SET_RTSP_PORT_FAILED | 12805 | Failed to set RTSP port |
| NETDEV_E_SET_ONVIF_PORT_FAILED | 12806 | Failed to set ONVIF port |
| NETDEV_E_SET_HTTPS_PORT_FAILED | 12807 | Failed to set HTTPS port |
| NETDEV_E_SET_HTTP_PORT_FAILED | 12808 | Failed to set HTTP port |
| NETDEV_E_TEST_EMAIL | 12809 | Failed to send test email |
| NETDEV_E_DDNS_INIT | 12810 | Initialize default value |
| NETDEV_E_DDNS_GOOD | 12811 | Update completed |
| NETDEV_E_DDNS_NOCHG | 12812 | Update completed, but IP remains unchanged |
| NETDEV_E_DDNS_BADAUTH | 12813 | Incorrect username or password |
| NETDEV_E_DDNS_DONATOR | 12814 | Special options require service fees similar to offline URLs |
| NETDEV_E_DDNS_NOTFQ | 12815 | Invalid host name format |
| NETDEV_E_DDNS_NOHOST | 12816 | No account for the host name |
| NETDEV_E_DDNS_NUMHOST | 12817 | Request immediate update for more than 20 hosts |
| NETDEV_E_DDNS_ABUSE | 12818 | Host name is forbidden for misuse |
| NETDEV_E_DDNS_BADAGENT | 12819 | Invalid format of HTTP request sent by agent |
| NETDEV_E_DDNS_GOOD_127 | 12820 | Unless the client requests to update 127.0.0.1, this message indicates an error in the HTTP request specification |
| NETDEV_E_DDNS_DNSERR | 12821 | DNS error, please contact technical support |
| NETDEV_E_DDNS_911 | 12822 | Service maintenance is in process |
| NETDEV_E_DDNS_REQ | 12823 | Failed to send DDNS request |
| NETDEV_E_DDNS_CHECK_KEY | 12824 | Incorrect check code |
| NETDEV_E_DDNS_BAD_NAME_FORMAT | 12825 | Invalid domain name format |
| NETDEV_E_DDNS_DOMAIN_CONFLICT | 12826 | Domain name conflict |
| NETDEV_E_DDNS_CLOSE | 12827 | Not returned by DDNS server, only used to close DDNS record status |
| NETDEV_E_CREATE_CERT_FAILD | 12828 | Failed to create certificate file |
| NETDEV_E_GET_CERT_FAILD | 12829 | Uploaded certificate file is invalid |
| NETDEV_E_TEST_EMAIL_CHECKSERVER_INFO | 12830 | Incorrect username or password for authentication server |
| NETDEV_E_TEST_EMAIL_COMM | 12831 | Failed to send mail, please check network and mail settings |
| NETDEV_E_TEST_EMAIL_TIMEOUT | 12832 | Mail sending timed out |
| NETDEV_E_CLOUD_REPEAT_USER | 12833 | Cloud account logged in |
| NETDEV_E_CLOUD_DEV_ADD_FAIL | 12834 | Failed to add cloud device info |
| NETDEV_E_CLOUD_ORG_NULL | 12835 | Cloud organization info is empty |
| NETDEV_E_GET_CLOUD_ORG | 12836 | Failed to get cloud info, please check network settings |
| NETDEV_E_NO_SUCH_CLOUD_USER | 12837 | Cloud account does not exist |
| NETDEV_E_NET_MAX | 13199 | Maximum error code of network module |
| NETDEV_E_CONF_FILE_INVALID | 13200 | Invalid configuration file |
| NETDEV_E_LOAD_XML_FAIL | 13201 | Failed to export XML file |
| NETDEV_E_SAVE_XML_FAIL | 13202 | Failed to create XML file |
| NETDEV_E_XML_ATTR_INEXIST | 13203 | XML attribute info does not exist |
| NETDEV_E_XML_SYNC_CFG_FAILED | 13204 | Error occurred while syncing configurations, used to save configurations synchronously during system restart |
| NETDEV_E_SIMPLE_RECOVER_CFG_FAILED | 13205 | Error occurred while restoring defaults |
| NETDEV_E_RECOVER_CFG_FAILED | 13206 | Error occurred while restoring factory defaults |
| NETDEV_E_CONF_FORBID_WRITE | 13207 | Writing configurations is forbidden |
| NETDEV_E_XML_MAX | 13599 | Max error code of XML module |
| NETDEV_E_PLAN_TEMPLATE_BUSY | 13600 | Time template is already in use |
| NETDEV_E_PLAN_TEMPLATE_NOT_EXIST | 13601 | Time template does not exist |
| NETDEV_E_PLAN_INVALID_HOLIDAY_TIME | 13602 | Invalid holiday time |
| NETDEV_E_PLAN_NO_PLAN | 13603 | No plan |
| NETDEV_E_PLAN_NOT_IN_PLAN | 13604 | Not in plan |
| NETDEV_E_PLAN_IN_PLAN | 13605 | In plan |
| NETDEV_E_PLAN_MAX | 13699 | Max error code of plan module |
| NETDEV_E_UPDATE_VERSION_FAIL | 13700 | Device upgrade, version number does not match |
| NETDEV_E_UPDATE_BOARDTYPE_NOT_MATCH | 13701 | Device upgrade, device type does not match |
| NETDEV_E_FILE_CHECK_FAILED | 13702 | File check failed |
| NETDEV_E_EXPORT_CFG_FAILED | 13703 | Error occurred while exporting configurations |
| NETDEV_E_IMPORT_CFG_FAILED | 13704 | Error occurred while importing configurations |
| NETDEV_E_UPDATE_OTHER_REASON | 13705 | Device upgrade, memory request failure or other failures |
| NETDEV_E_MAINTAIN_EXPORT_RUNNING | 13706 | Exporting maintenance info |
| NETDEV_E_MT_MAX | 13799 | Max error code of MT module |
| NETDEV_E_LINKAGE_GUARD_PRO_ALREADY_EXIT | 13800 | Arming schedule already exists |
| NETDEV_E_LINKAGE_GUARD_PRO_NOT_EXIST | 13801 | Specified arming schedule does not exist |
| NETDEV_E_LINKAGE_LINK_RULE_NOT_EXIST | 13802 | Specified link rule does not exist |
| NETDEV_E_LINKAGE_MAX | 13899 | Max error code of Linkage module |
| NETDEV_E_ALARM_RESTRAIN | 13900 | Alarm suppression |
| NETDEV_E_EMAP_PIC_MAX_NUM | 15000 | Maximum image number, no more images can be added |
| NETDEV_E_EMAP_DIR_EXIST | 15001 | Root directory already exists in e-map |
| NETDEV_E_EMAP_MAX | 15099 | Max error code of e-map module |
| NETDEV_E_PACS_PERSON_NO_EXIST | 17000 | Access control person does not exist |
| NETDEV_E_PACS_PERSON_EXIST | 17001 | Access control person already exists |
| NETDEV_E_PACS_PERSON_CARD_EXIST | 17002 | Access control card already exists |
| NETDEV_E_PACS_PERSON_CARD_NO_EXIST | 17003 | Access control card does not exist |
| NETDEV_E_PACS_PERSON_CARD_REACH_LIMIT | 17004 | Number of access control cards reached the upper limit |
| NETDEV_E_PACS_PERMISSION_EXIT_PERSON | 17011 | Cannot delete permission groups containing persons |
| NETDEV_E_PACS_BLACKLIST_PERSON_EXIST | 17012 | Blocklist person already exists |
| NETDEV_E_PACS_BLACKLIST_NOT_ADD | 17013 | Cannot add blacklist person |
| NETDEV_E_PACS_MAX | 17999 | Max error code of PACS module |
| NETDEV_E_FACE_BD_NO_MEMBER | 19000 | No member in face library |
| NETDEV_E_FACE_GUARD_NO_EXIST | 19001 | Monitoring task does not exist |
| NETDEV_E_FACE_ALARM_LOG_NO_EXIST | 19002 | Face recognition alarm record does not exist |
| NETDEV_E_SMART_GET_FACE_IMAGE_FAIL | 19003 | Failed to get face snapshot |
| NETDEV_E_NO_UID_AVAILABLE | 19004 | No ID available |
| NETDEV_E_FACE_LIB_COUNT_MAX | 19005 | Number of face libraries reached the upper limit |
| NETDEV_E_FACE_GUARD_COUNT_MAX | 19006 | Number of monitoring tasks of the face library reached the upper limit |
| NETDEV_E_FACE_MEMBER_COUNT_MAX | 19007 | Number of members of the face library reached the upper limit |
| NETDEV_E_FACE_IDENTI_NUM_EXIST | 19008 | ID number already exists |
| NETDEV_E_FACE_FEAT_VERTION_UNMATCH | 19009 | Device version does not match |
| NETDEV_E_VEHICLE_GUARD_NO_EXIST | 19010 | Vehicle monitoring task does not exist |
| NETDEV_E_SLAVE_PLATE_IMAGE_GET_FAIL | 19011 | Failed to get plate image from slave server |
| NETDEV_E_VEHICLE_LOG_NO_EXIST | 19012 | Vehicle record does not exist |
| NETDEV_E_GET_SLAVE_PIC_FAIL | 19013 | Failed to get image from slave server |
| NETDEV_E_GUARD_DEV_VIID_OFFLINE | 19014 | Monitoring device VIID offline |
| NETDEV_E_GUARD_GUARD_DEV_NOW | 19015 | Deploying on device |
| NETDEV_E_GUARD_GUARD_NOT_ENABLE | 19020 | Monitoring task is disabled |
| NETDEV_E_GUARD_GUARD_NOW | 19021 | Monitoring |
| NETDEV_E_GUARD_DIFFERENT_IMAGE_PROTOCOL | 19022 | Each monitoring task can only use devices that are connected via the same image protocol for monitoring |
| NETDEV_E_ROOT_LIB_HAS_MONITORED_MEMBER | 19023 | Root library includes monitored member(s) |
| NETDEV_E_SMART_MAX | 19999 | Max error code of smart module |
| NETDEV_E_VIID_DISPOSE_DEV_BUSY | 25006 | VIID deployment error - device busy |
| NETDEV_E_VIID_DISPOSE_DATABASE_FULL | 25007 | VIID deployment error - number of face libraries reached the upper limit |
| NETDEV_E_VIID_DISPOSE_MEMBER_FULL | 25008 | VIID deployment error - number of face members reached the upper limit |
| NETDEV_E_VIID_DISPOSE_NO_SMARTBAR | 25009 | VIID deployment error - no smart bar on device |
| NETDEV_E_VIID_DISPOSE_VIID_OFFLINE | 25010 | VIID deployment error - device is not connected to VIID |
| NETDEV_E_CLOUD_REGISTER_BY_OTHER | 20000 | User has no permission |
| NETDEV_E_CLOUD_CHL_NOT_EXIST | 20001 | Channel does not exist |
| NETDEV_E_CLOUD_SREAMID_NOT_EXIST | 20002 | Stream ID does not exist |
| NETDEV_E_CLOUD_BITRATE_UNEXPECTED | 20003 | Requested bit rate is too high |
| NETDEV_E_CLOUD_LIVEID_NOT_EXIST | 20004 | Live ID does not exist |
| NETDEV_E_CLOUD_PROTO_TYPE_ILLEGAL | 20005 | Requested stream protocol is not supported |
| NETDEV_E_CLOUD_CLIENTID_NOT_EXIST | 20006 | Client ID does not exist |
| NETDEV_E_CLOUD_REQUESTID_NOT_EXIST | 20007 | RequestID does not exist |
| NETDEV_E_CLOUD_VISITOR_TOTAL_UNEXPECTED | 20008 | Too many online users |
| NETDEV_E_GET_RTMP_URL_FAIL | 20009 | Failed to get RTMP URL |
| NETDEV_E_GET_PUSH_STREAM_STATE_FAIL | 20010 | Failed to get stream push status |
| NETDEV_E_START_PULL_STREAM_FAIL | 20011 | Failed to start pulling stream |
| NETDEV_E_GET_CLOUD_DEV_LIST_FAIL | 20012 | Failed to get cloud device list |
| NETDEV_E_GET_DEV_INFO_FAILED | 20013 | Failed to get cloud device info |
| NETDEV_E_GET_NAT_TYPE_FAIL | 20014 | Failed to get NAT type |
| NETDEV_E_MODIFT_CLOUD_DEV_NAME_FAIL | 20015 | Failed to update cloud device name |
| NETDEV_E_DELETE_CLOUD_DEVICE_FAIL | 20016 | Failed to delete cloud device |
| NETDEV_E_STOP_PULL_STREAM_FAIL | 20017 | Failed to stop pulling stream |
| NETDEV_E_KEEP_ALIVE_PULL_STREAM_FAIL | 20018 | Failed to keep pull stream alive |
| NETDEV_E_KEEP_ALIVE_LIST_EMPTY | 20019 | Keep-alive list is empty |
| NETDEV_E_GET_LIVE_ID_FAIL | 20020 | Failed to get live ID |
| NETDEV_E_CLOUD_INVALID_FORMAT | 20021 | Invalid format |
| NETDEV_E_CLOUD_NOT_REGISTER | 20022 | Device is not registered |
| NETDEV_E_CLOUD_INVALID_REG_CODE | 20023 | Register code does not exist |
| NETDEV_E_CLOUD_NOT_SHARE_TO_SELF | 20024 | Cannot share to yourself |
| NETDEV_E_ADD_CLOUD_DEVICE_FAIL | 20025 | Failed to add cloud device |
| NETDEV_E_GET_CLOUD_DDNS_FAIL | 20026 | Failed to get DDNS info |
| NETDEV_E_GET_CLOUD_ORG_LIST_FAIL | 20027 | Failed to get cloud organization list |
| NETDEV_E_FIND_SHARE_DEVLIST_ERROR | 20028 | Failed to get shared device list |
| NETDEV_E_START_DEVICE_SHARE_FAIL | 20029 | Failed to start device sharing |
| NETDEV_E_STOP_DEVICE_SHARE_FAIL | 20030 | Failed to stop device sharing |
| NETDEV_E_GET_SHARE_RECORD_INFO_FAIL | 20031 | Failed to get device sharing records |
| NETDEV_E_GET_CLOUD_DEV_POPULATION | 20032 | Failed to get device statistics of a cloud account |
| NETDEV_E_ADD_CLOUD_ORG_FAIL | 20033 | Failed to add cloud organization |
| NETDEV_E_MODIFY_CLOUD_ORG_FAIL | 20034 | Failed to modify cloud organization |
| NETDEV_E_DELETE_CLOUD_ORG_FAIL | 20035 | Failed to delete cloud organization |
| NETDEV_E_ADD_DEV_TO_CLOUD_ORG_FAIL | 20036 | Failed to add device to cloud organization |
| NETDEV_E_GET_CLOUD_DEV_CHANNEL_LIST_FAIL | 20037 | Failed to get cloud device channel |
| NETDEV_E_GET_CLOUD_VIDEO_TIME_FAIL | 20038 | Failed to get cloud video time info |
| NETDEV_E_GET_CLOUD_VIDEO_M3U8_FAIL | 20039 | Failed to get cloud video M3u8 info |
| NETDEV_E_SNAPSHOT_PIC_FAILED | 27000 | Image storage exception |
| NETDEV_E_SNAPSHOT_QRY_PIC_FAILED | 27001 | Image query exception |
| NETDEV_E_SNAPSHOT_MAX | 27999 | Max error code of snapshot module |

# Data Structure

## NETDEV_DEVICE_INFO_S

/* Device info */

public class NETDEV_CLOUD_DEV_LOGIN_S {

public int dwDevType;             /* Device type */

public int dwAlarmInPortNum;      /* Number of alarm inputs */

public int dwAlarmOutPortNum;     /* Number of alarm outputs */

public int dwChannelNum ;         /* Number of channels */

}

## NETDEV_FILECOND_S

/* Recording query */

public class NETDEV_FILECOND_S{

public String    szFileName;          /* Recording file name */

public int   dwChannelID;             /* Channel ID */

public int   dwFileType;              /* Recording file type*/

public long   tBeginTime;             /* Start time */

public long   tEndTime;               /* End time */

}

## NETDEV_FINDDATA_S

/* Recording query info */

public class NETDEV_FINDDATA_S {

public String    szFileName;     /* Recording file name */

public long   tBeginTime;        /* Start time */

public long   tEndTime;          /* End time */

public int   byFileType;         /* Recording file type */

}

## NETDEV_PLAYBACKCONTROL_S

/* Playback speed control */

public class NETDEV_PLAYBACKCONTROL_S {

public int enSpeed;              /* Playback speed */

}

## NETDEV_PREVIEWINFO_S

/* Live view parameters */

public class NETDEV_PREVIEWINFO_S {

public int dwChannelID;             /* Channel ID  */

public int dwStreamIndex;           /* Stream type */

public int dwLinkMode;              /* Transport protocol */

public int dwFluency;                /* Video fluency priority*/

public int dwStreamMode;            /* Stream mode*/

public int dwLiveMode;              /* Stream startup mode */

public int dwDisTributeCloud;         /* Distribution capacity */

}

Related enumerations

dwStreamIndex

| Value | Description |
| --- | --- |
| 0 | Main stream |
| 1 | Sub stream |
| 2 | Third stream |

dwLinkMode

| Value | Description |
| --- | --- |
| 0 | UDP |
| 1 | TCP |

dwFluency

| Value | Description |
| --- | --- |
| 0 | Real-time priority |
| 1 | Fluency priority |
| 2 | Balanced |
| 4 | RTMP fluency priority |

dwStreamMode

| Value | Description |
| --- | --- |
| 0x0000 | Audio+video |
| 0x8000 | Only video |

dwLinkMode

| Value | Description |
| --- | --- |
| 0 | Assigned by server |
| 1 | Distribute by force |

dwDisTributeCloud

| Value | Description |
| --- | --- |
| 0 | Disable CDN |
| 1 | Enable CDN by JSYun |
| 2 | Enable CDN by EZCloud |
| 3 | Enable CDN by Tencent Cloud |

## NETDEV_VIDEO_CHL_DETAIL_INFO_S

/* Video channel details */

public class NETDEV_VIDEO_CHL_DETAIL_INFO_S {

public int dwChannelID;         /* Channel ID */

public int bPtzSupported;        /* Whether PTZ is supported */

public int enStatus;             /* Channel status */

public int dwStreamNum;          /* Number of streams */

}

## NETDEV_VIDEO_CHL_DETAIL_INFO_EX_S

/* Video channel details */

public class NETDEV_VIDEO_CHL_DETAIL_INFO_EX_S {

public int dwChannelID;        /* Channel ID  */

public int bPtzSupported;      /* Whether PTZ is supported */

public int enStatus;           /* Channel status, see NETDEV_CHANNEL_STATUS_E */

public int dwStreamNum;   /* Number of streams, the value is invalid when enStatus is NETDEV_CHL_STATUS_UNBIND */

public int enChannelType;       /* Channel type (note: effective to hybrid NVR only) */

public int enVideoFormat;  /* Video input format, the value is invalid when ChannelType is NETDEV_CHL_TYPE_DIGITAL (note: effective to hybrid NVR only) */

public int enAddressType;      /* IP address type */

public String szIPAddr;        /* IP address*/

public int dwPort;              /* Port number */

public String szChnName;       /* Channel name */

public int allowDistribution;  /* Whether CDN is supported*/

public int dwDeviceType;   /* Device type, see NETDEV_CHANNEL_CAMERA_TYPE_E */

public String szManufacturer;  /* Manufacturer, range [0,31] */

public String szDeviceModel;   /* Device model, range [0,31] */

public class NETDEV_CHANNEL_CAMERA_TYPE_E

{

public static final int NETDEV_CHL_CAMERA_TYPE_FIXED        = 0;        /* Fixed camera */

public static final int NETDEV_CHL_CAMERA_TYPE_PTZ          = 1;        /* PTZ camera */

public static final int NETDEV_CHL_CAMERA_TYPE_FISH_EYE     = 2;        /* Fisheye camera (with fisheye lens and dewarping functionality) */

public static final int NETDEV_CHL_CAMERA_TYPE_WIDE_ANGLE   = 3;        /* Ultra-wide camera (with fisheye lens or wide-angle lens but without dewarping functionality) */

public static final int NETDEV_CHL_CAMERA_TYPE_INVALID      = 0xff;

}

public class NETDEV_CHANNEL_STATUS_E

{

public static final int NETDEV_CHL_STATUS_OFFLINE   = 0;            /* Channel offline */

public static final int NETDEV_CHL_STATUS_ONLINE    = 1;             /* Channel online */

public static final int NETDEV_CHL_STATUS_UNBIND    = 2;             /* Channel unbind */

public static final int NETDEV_CHL_STATUS_INVALID   = 0xff;

}

}

## NETDEV_TIME_S

/* Time info */

public class NETDEV_TIME_S {

public int dwYear;                     /*   Year */

public int dwMonth;                      /*  Month */

public int dwDay;                        /*   Day */

public int dwHour;                       /*   Hour */

public int dwMinute;                     /*   Minute */

public int dwSecond;                     /*   Second */

}

## NETDEV_MONTH_INFO_S

/* Configurations of querying recording status by month */

public class NETDEV_MONTH_INFO_S {

public int udwYear;              /* Year */

public int udwMonth;             /* Month */

public int udwPosition;          /* Recording query location, default should be set to 1*/

}

## NETDEV_MONTH_STATUS_S

/* Recording status */

public class NETDEV_MONTH_STATUS_S {

int udwDayNumInMonth;               /*Number of days in a month*/

int szVideoStatus[] = new int[30]; /* Not in use */

}

## NETDEV_DEVICE_BASICINFO_S

/* Basic device info */

public class NETDEV_DEVICE_BASICINFO_S {

public String szDevModel;                     /* Device model */

public String szSerialNum;                    /* Hardware serial number */

public String szFirmwareVersion;              /* Software version number */

public String szMacAddress;                   /* IPv4 MAC address */

public String szDeviceName;                   /* Device name */

}

## NETDEV_PLAYBACKCOND_S

/* Recording playback and download parameters */

public class NETDEV_PLAYBACKCOND_S {

public int   	dwChannelID;                /* Channel ID */

public long 	tBeginTime;                 /* Start time */

public long   	tEndTime;                   /* End time */

public int   	dwLinkMode;                 /* Transport protocol */

public int  	hPlayWnd;                   /* Playback window handle */

public int   	dwFileType;                 /* Recording type */

public int   	dwDownloadSpeed;            /* Download speed */

public int      dwPlaySpeed;				/* Playback speed */

}

Related enumerations:

dwLinkMode

| Value | Description |
| --- | --- |
| 0 | UDP |
| 1 | TCP |

dwFileType

| Value | Description |
| --- | --- |
| 0 | Normal recording |
| 4 | Motion detection recording |
| 5 | Digital input recording |
| 7 | Video loss recording |

dwDownloadSpeed

| Value | Description |
| --- | --- |
| 0 | 1x |
| 1 | 2x |
| 2 | 4x |
| 3 | 8x |
| 4 | 2x I frame |
| 5 | 4x I frame |
| 6 | 8x I frame |
| 7 | 2/1x |
| 8 | 40x |

dwPlaySpeed

| 0 | 16x rewind |
| --- | --- |
| 1 | 8x rewind |
| 2 | 4x rewind |
| 3 | 2x rewind |
| 4 | 1x rewind |
| 5 | 1/2x rewind |
| 6 | 1/4x rewind |
| 7 | 1/4x play |
| 8 | 1/2x play |
| 9 | 1x forward |
| 10 | 2x forward |
| 11 | 4x forward |
| 12 | 8x forward |
| 13 | 16x forward |
| 14 | 2x forward (I frame) |
| 15 | 4x forward (I frame) |
| 16 | 8x forward (I frame) |
| 17 | 16x forward (I frame) |
| 18 | 2x rewind (I frame) |
| 19 | 4x rewind (I frame) |
| 20 | 8x rewind (I frame) |
| 21 | 16x rewind (I frame) |
| 22 | Smart playback |
| 23 | Forward frame by frame |
| 24 | Rewind frame by frame |
| 25 | 40x forward |
| 26 | 32x selected-frame forward |
| 27 | 32x selected-frame rewind |
| 28 | 64x selected-frame forward |
| 29 | 64x selected-frame rewind |
| 30 | 128x selected-frame forward |
| 31 | 128x selected-frame rewind |
| 32 | 256x selected-frame forward |
| 33 | 256x selected-frame rewind |
| 34 | 32x forward |
| 35 | 32x rewind |

## NETDEV_ALARMINFO_S

/* Alarm info */

public class NETDEV_ALARMINFO_S {

public int mUserID;	      /* User ID for NVR */

public int mChannelID;	  /* Channel ID for NVR */

public int mAlarmType;	  /* Alarm type*/

public int mAlarmTime;	  /* Alarm time */

public String mStrName;  /* Alarm source name, alarm input/output name */

}

## NETDEV_AUDIO_SAMPLE_PARAM_S

/* Audio parameters */

public class NETDEV_AUDIO_SAMPLE_PARAM_S {

public int dwChannels;         /* Number of sound tracks, 1 for mono, 2 for stereo */

public int dwSampleRate;       /* Sampling rate */

public int enSampleFormat;     /* Width, see NETDEV_AUDIO_SAMPLE_FORMAT_E */

public class NETDEV_AUDIO_SAMPLE_FORMAT_E{

public static final int NETDEV_AUDIO_SAMPLE_FMT_NONE = -1;

public static final int NETDEV_AUDIO_SAMPLE_FMT_U8  = 0;         /* Unsigned integer */

public static final int NETDEV_AUDIO_SAMPLE_FMT_S16 = 1;        /* Signed integer */

public static final int NETDEV_AUDIO_SAMPLE_FMT_S32 = 2;        /* Signed integer */

public static final int NETDEV_AUDIO_SAMPLE_FMT_FLT = 3;        /* Float */

public static final int NETDEV_AUDIO_SAMPLE_FMT_DBL = 4;       /* Double */

}

}

## NETDEV_PTZ_ABSOLUTE_MOVE_S

/* Absolute coordinates movement parameters */

public class NETDEV_PTZ_ABSOLUTE_MOVE_S {

public float fPanTiltX;			/* Absolute X coordinate */

public float fPanTiltY;             /* Absolute Y coordinate */

public float fZoomX;                /* Absolute zoom ratio */

}

## NETDEV_PTZ_ALLPRETSET_S

/* All PTZ presets */

public class NETDEV_PTZ_ALLPRETSET_S {

public int size ;                         			/* Total number of presets */

public int[] presetID = new int[256];  			/* Preset ID */

public String[] pretsetName = new String[256];	/* Preset name */

}

## NETDEV_PTZ_PRESET_S

/* PTZ preset info */

public class NETDEV_PTZ_PRESET_S {

public int presetID;				/* Preset ID */

public String presetName;			/* Preset name */

}

## NETDEV_PTZ_STATUS_S

/* PTZ status */

public class NETDEV_PTZ_STATUS_S {

public float fPanTiltX;			 /* Absolute X coordinate */

public float fPanTiltY;              /* Absolute Y coordinate */

public float fZoomX;                /* Absolute zoom ratio */

public int enPanTiltStatus;         /* PTZ status 0: Idle 1: Moving */

public int enZoomStatus;            /* Zoom status 0: Idle 1: Zooming */

}

## NETDEV_DEVICE_LOGIN_INFO_S

/* Login info */

public class NETDEV_DEVICE_LOGIN_INFO_S {

public String szIPAddr;                       /* IP/domain name */

public int dwPort;                            /* Port number */

public String szUserName;                    /* Username */

public String szPassword;                    /* Password */

public int dwLoginProto;                      /* Login protocol, see NETDEV_LOGIN_PROTO_E */

public int dwDeviceType;                      /* Device type, see NETDEV_DEVICE_TYPE_E */

public class NETDEV_LOGIN_PROTO_E

{

public static final int NETDEV_LOGIN_PROTO_ONVIF             = 0;           /* Connect by ONVIF */

public static final int NETDEV_LOGIN_PROTO_PRIVATE           = 1;            /* Connect by private protocol */

}

public class NETDEV_DEVICE_TYPE_E

{

public static final int NETDEV_DTYPE_UNKNOWN                        = 0;            /* Unknown type */

public static final int NETDEV_DTYPE_IPC                            = 1;            /* IPC range */

public static final int NETDEV_DTYPE_IPC_FISHEYE                 = 2;            /* Fisheye camera (with fisheye or wide-angle lens and dewarping functionality) */

public static final int NETDEV_DTYPE_IPC_ECONOMIC_FISHEYE       = 3;            /* Fisheye camera (with fisheye or wide-angle lens but without dewarping functionality) */

public static final int NETDEV_DTYPE_NVR                            = 101;          /* NVR range */

public static final int NETDEV_DTYPE_NVR_BACKUP                    = 102;          /* NVR backup server */

public static final int NETDEV_DTYPE_HNVR                           = 103;          /* Hybrid NVR */

public static final int NETDEV_DTYPE_DC                             = 201;          /* DC range */

public static final int NETDEV_DTYPE_DC_ADU                         = 202;          /* ADU range */

public static final int NETDEV_DTYPE_EC                             = 301;          /* EC range */

public static final int NETDEV_DTYPE_VMS                            = 501;          /* VMS range */

public static final int NETDEV_DTYPE_FG                             = 601;          /* FG range */

public static final int NETDEV_DTYPE_INVALID                    = 0xFFFF;        /* Invalid value */

}

}

## NETDEV_SELOG_INFO_S

/* Secure login info */

public class NETDEV_SELOG_INFO_S {

public int  dwSELogCount;   /* Number of secure logins */

public int  dwSELogTime;     /* Secure login time */

}

## NETDEV_ORG_FIND_COND_S

/* Criteria for querying organization list */

public class NETDEV_ORG_FIND_COND_S {

public int udwOrgType;              /* Organization type, see NETDEV_ORG_TYPE_E */

public int udwRootOrgID;            /* Root organization ID */

public int udwFindType;           /* Query mode, see NETDEV_ORG_FIND_MODE_E */

public class NETDEV_ORG_TYPE_E

{

public static final int NETDEV_ORG_TYPE_GENERAL = 0;    /* General organization */

public static final int NETDEV_ORG_TYPE_CLOUD = 1;      /* Cloud organization */

public static final int NETDEV_ORG_TYPE_VIRTUAL = 2;    /* Logical organization */

public static final int NETDEV_ORG_TYPE_FAVORITES = 3; /* Favorites */

public static final int NETDEV_ORG_TYPE_DOMAIN = 4; /* Domain */

public static final int NETDEV_ORG_TYPE_DOORGROUP= 5; /* Door group */

public static final int NETDEV_ORG_TYPE_DEPT= 6;   /* Department */

public static final int NETDEV_ORG_TYPE_INVALID = 0XFF; /* Invalid value */

}

public class NETDEV_ORG_FIND_MODE_E

{

public static final int NETDEV_ORG_FIND_MODE_ID = 0;   /* Organization ID */

public static final int NETDEV_ORG_FIND_MODE_TYPE = 1;  /* Organization type */

public static final int NETDEV_ORG_FIND_MODE_INVALID = 0XFF;  /* Invalid value */

}

}

## NETDEV_ORG_INFO_S

/* Organization info */

public class NETDEV_ORG_INFO_S {

public int dwOrgID;                   /* Organization ID */

public int dwParentID;                /* Parent node ID */

public int dwType;                    /* Type, see NETDEV_ORG_TYPE_E */

public String  szNodeName;            /* Node name */

public String  szDesc;                /* Remarks */

public class NETDEV_ORG_TYPE_E

{

public static final int NETDEV_ORG_TYPE_GENERAL = 0; /* General organization */

public static final int NETDEV_ORG_TYPE_CLOUD   = 1; /* Cloud organization */

public static final int NETDEV_ORG_TYPE_VIRTUAL = 2; /* Logical organization */

public static final int NETDEV_ORG_TYPE_FAVORITES = 3; /* Favorites */

public static final int NETDEV_ORG_TYPE_DOMAIN   = 4; /* Domain */

public static final int NETDEV_ORG_TYPE_DOORGROUP = 5; /* Door group */

public static final int NETDEV_ORG_TYPE_DEPT       = 6;  /* Department */

public static final int NETDEV_ORG_TYPE_INVALID  = 0XFF; /* Invalid value */

}

}

## NETDEV_ORG_CHN_INFO_S

/* Organization channel info */

public class NETDEV_ORG_CHN_INFO_S {

public int dwOrgID;                              /* Organization ID */

public int dwChannelID;                          /* Channel ID */

public String szChnName;                         /* Channel name */

public int dwChnIndex;                           /* Channel index */

public int dwChnType;                            /* Channel type */

public int dwChnStatus;                          /* Channel status */

public int dwDevID;                              /* Device ID */

public String szDevName;                         /* Device name */

}

## NETDEV_DEV_BASIC_INFO_S

/* Basic device info */

public class NETDEV_DEV_BASIC_INFO_S {

public NETDEV_IPADDR_INFO_S stDevAddr;               /* Device IP address info */

public NETDEV_USER_SIMPLE_INFO_S stDevUserInfo;      /* Device username and password */

public String szDevName;                              /* Device name */

public String szDevDesc;                              /* Device description */

public String szDevModel;                             /* Device model */

public int dwDevID;                                   /* Device ID */

public int dwDevStatus;                               /* Device status, see # NETDEV_DEVICE_STATUS_E */

public int dwDevType;                                 /* Device type, see # NETDEV_DEVICE_MAIN_TYPE_E */

public int dwDevSubType;                              /* Device subtype, see # NETDEV_DEVICE_SUB_TYPE_E */

public int dwOrgID;                                   /* Organization ID */

public int dwAccessProtocol;                         /* Connection protocol */

public int dwAccessMode;                             /* Connection mode, see # NETDEV_DEVICE_ACCESS_MODE_E*/

public int dwServerID;                             /* Server ID */

public int dwAudioResID;                           /* Audio channel ID */

public String  szVIIDCode;                 /* VIID code, valid only for LPR camera or face recognition camera */

public int dwVIIDStatus;                           /* VIID status, used to indicate whether the device is connected by video&image database protocol, 0: offline, 1: online */

public String szSerialNum;                         /* Device serial number */

public String szSoftVersion;                       /* Software version number */

public String szMacAddr;                           /* MAC address */

public int dwStoreStatus;                          /* Device storage status 0: Idle 1: Not formatted 2: Formatting 3: Operating */

public NETDEV_ONVIF_INFO_S stOnvifInfo;          /* ONVIF info */

public NETDEV_GBINFO_S stGBInfo;                 /* GB/T28181 info, required only when AccessProtocol=3 */

public NETDEV_DEV_BASIC_INFO_S() {

stDevAddr = new NETDEV_IPADDR_INFO_S();

stDevUserInfo = new NETDEV_USER_SIMPLE_INFO_S();

stOnvifInfo = new NETDEV_ONVIF_INFO_S();

stGBInfo = new NETDEV_GBINFO_S();

}

public class NETDEV_DEVICE_STATUS_E

{

public static final int NETDEV_DEV_STATUS_OFFLINE         = 0;                /* Offline */

public static final int NETDEV_DEV_STATUS_ONLINE          = 1;                /* Online */

public static final int NETDEV_DEV_STATUS_CONNECTING      = 2;                /* Connecting */

public static final int NETDEV_DEV_STATUS_PWD_ERROR       = 3;                /* Incorrect username or password */

public static final int NETDEV_DEV_STATUS_NOT_SUPPORT     = 4;                /* Unsupported device */

public static final int NETDEV_DEV_STATUS_TIMEOUT          = 5;                /* Device connection timed out */

public static final int NETDEV_DEV_STATUS_WEAK_PWD_ERROR  = 6;                /* Weak password error for remote user */

public static final int NETDEV_DEV_STATUS_NO_DYNAMIC_PWD  = 7;                /* Device does not support dynamic password */

public static final int NETDEV_DEV_STATUS_INVALID         = 0XFF;              /* Invalid value */

}

public class NETDEV_DEVICE_MAIN_TYPE_E

{

public static final int NETDEV_DTYPE_MAIN_ENCODE            = 0;                /* Encoding device */

public static final int NETDEV_DTYPE_MAIN_DECODE            = 1;                /* Decoding device */

public static final int NETDEV_DTYPE_MAIN_VMS               = 2;                /* VMS */

public static final int NETDEV_DTYPE_MAIN_DA                = 3;                /* Agent */

public static final int NETDEV_DTYPE_MAIN_CLOUD             = 4;                /* Cloud encoding device */

public static final int NETDEV_DTYPE_MAIN_BAYONET           = 5;                /* License plate or face recognition device */

public static final int NETDEV_DTYPE_MAIN_ACS               = 6;                /* Access control device */

public static final int NETDEV_DTYPE_MAIN_ALARMHOST         = 7;                /* Alarm control device */

public static final int NETDEV_DTYPE_MAIN_UNKNOWN           = 0XFF;              /* Unknown device */

}

public class NETDEV_DEVICE_SUB_TYPE_E

{

public static final int NETDEV_DTYPE_SUB_NVR                = 0;                /* NVR */

public static final int NETDEV_DTYPE_SUB_IPC                = 1;                /* IPC */

public static final int NETDEV_DTYPE_SUB_DC_INNER           = 2;                /* Internal DC */

public static final int NETDEV_DTYPE_SUB_DC_EXT             = 3;                /* External DC */

public static final int NETDEV_DTYPE_SUB_EC                 = 4;                /* EC */

public static final int NETDEV_DTYPE_SUB_VMS                = 5;                /* VMS */

public static final int NETDEV_DTYPE_SUB_DA                 = 6;                /* Agent */

public static final int NETDEV_DTYPE_SUB_ADU                = 7;                /* ADU */

public static final int NETDEV_DTYPE_SUB_FISHEYE_IPC        = 8;                /* Fisheye camera */

public static final int NETDEV_DTYPE_SUB_DECARD             = 9;                /* Decoding card */

public static final int NETDEV_DTYPE_SUB_FACE_IPC           = 10;               /* Face recognition camera */

public static final int NETDEV_DTYPE_SUB_ALARMHOST          = 11;               /* Alarm control device */

public static final int NETDEV_DTYPE_SUB_ACS                = 12;               /* Access control device */

public static final int NETDEV_DTYPE_SUB_UNKNOW             = 0xFF;              /* Unknown device */

}

public class NETDEV_DEVICE_ACCESS_MODE_E

{

public static final int NETDEV_DEV_ACCESS_MODE_IP_DOMAIN            = 0;                /* Connect by IP or domain name */

public static final int NETDEV_DEV_ACCESS_MODE_P2P                  = 1;                /* Connect by P2P */

public static final int NETDEV_DEV_ACCESS_MODE_UNP                  = 2;                /* Connect by UNP */

public static final int NETDEV_DEV_ACCESS_MODE_INVALID              = 0XFF;              /* Invalid value */

}

}

## NETDEV_DEV_CHN_ENCODE_INFO_S

/* Organization channel info */

public class NETDEV_DEV_CHN_ENCODE_INFO_S {

public NETDEV_DEV_CHN_BASE_INFO_S  stChnBaseInfo;  /* Basic channel info */

public int                         dwMaxStream;    /* Max number of streams supported */

public int                         bSupportPTZ;    /* Whether PTZ is supported */

public int                         dwAudioResID;   /* Audio resource ID */

public NETDEV_DEV_CHN_ENCODE_INFO_S() {

stChnBaseInfo = new NETDEV_DEV_CHN_BASE_INFO_S();

}

}

## NETDEV_MULTI_TRAFFIC_STATISTICS_COND_S

/*People flow counting struct*/

public class NETDEV_MULTI_TRAFFIC_STATISTICS_COND_S {

public NETDEV_OPERATE_LIST_S    stChannelIDs;               /* Channel ID list */

public int                     udwStatisticsType;          /* See NETDEV_TRAFFIC_STATISTICS_TYPE_E */

public int                     udwFormType;                /* See NETDEV_TRAFFIC_STATIC_FORM_TYPE_E */

public long                    tBeginTime;                 /* Start time */

public long                    tEndTime;                   /* End time */

public NETDEV_MULTI_TRAFFIC_STATISTICS_COND_S() {

stChannelIDs = new NETDEV_OPERATE_LIST_S();

}

public class NETDEV_TRAFFIC_STATISTICS_TYPE_E

{

public static final int NETDEV_TRAFFIC_STATISTICS_TYPE_IN   = 0;        /* Statistics type : Enter */

public static final int NETDEV_TRAFFIC_STATISTICS_TYPE_OUT  = 1;        /* Statistics type : Exit */

public static final int NETDEV_TRAFFIC_STATISTICS_TYPE_ALL  = 2;        /* Statistics type : All */

public static final int NETDEV_TRAFFIC_STATISTICS_TYPE_BUTT = 3;

}

public class NETDEV_TRAFFIC_STATIC_FORM_TYPE_E

{

public static final int NETDEV_TRAFFIC_STAT_FORM_BY_MINUTE = 0;                  /* By minute*/

public static final int NETDEV_TRAFFIC_STAT_FORM_BY_HOUR = 1;                    /* By hour*/

public static final int NETDEV_TRAFFIC_STAT_FORM_BY_DAY = 2;                     /* By day*/

public static final int NETDEV_TRAFFIC_STAT_FORM_BY_MONTH = 3;                   /* By month*/

public static final int NETDEV_TRAFFIC_STAT_FORM_BY_BUTT = 4;

}

}

## NETDEV_TRAFFIC_STATISTICS_INFO_S

/*People flow query info*/

public class NETDEV_TRAFFIC_STATISTICS_INFO_S {

public int   bIsSuccess;                                    /* Whether query is successful */

public int   dwChannelID;                                   /* Channel ID */

public int   udwSize;                                       /* Number of time periods */

public int[] audwEnterCount;                                /* Entry count */

public int[] audwExitCount;                                 /* Exit count */

public NETDEV_TRAFFIC_STATISTICS_INFO_S() {

audwEnterCount = new int[60];

audwExitCount = new int[60];

}

}

## AlarmCallBack_PF

/* Alarm callback */

public class AlarmCallBack_PF {

MapAlarmType[] alarmTypeMap = {

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_MOVE_DETECT,"move detect"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VIDEO_LOST,"video lost"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VIDEO_TAMPER_DETECT,"video tamper detect"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_INPUT_SWITCH,"input switch"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_TEMPERATURE_HIGH,"temperature high"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_TEMPERATURE_LOW,"temperature low"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_AUDIO_DETECT,"audio detect"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_ABNORMAL,"disk abnormal"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_OFFLINE,"disk offline"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_ONLINE,"disk online"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_STORAGE_WILL_FULL,"disk storage will pull"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_STORAGE_IS_FULL,"disk storage is full"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_DISABLED,"disk raid disabled"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_DEGRADED,"disk raid degraded"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_RECOVERED,"disk raid recovered"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_ONLINE,"device online"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_OFFLINE,"device offline"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_REBOOT,"device reboot"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_SERVICE_REBOOT,"device service reboot"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_DELETE_CHL,"device delete chl"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_NET_FAILED,"network failed"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_NET_TIMEOUT,"network timeout"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SHAKE_FAILED,"shake failed"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STREAMNUM_FULL,"stream num full"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STREAM_THIRDSTOP,"stream third stop"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_FILE_END,"file end"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_RTMP_CONNECT_FAIL,"rtmp connect fail"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_RTMP_INIT_FAIL,"rtmp init fail"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STOR_ERR,"store error"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STOR_DISOBEY_PLAN,"store error "),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_ERROR,"disk error"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ILLEGAL_ACCESS,"illegal access"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_LINE_CROSS,"line cross"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_OBJECTS_INSIDE,"inside"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_FACE_RECOGNIZE,"face recognize"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_IMAGE_BLURRY,"image blurry"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SCENE_CHANGE,"scene change"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_TRACK,"smart track"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_LOITERING_DETECTOR,"loitering detector"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ALLTIME_FLAG_END,"all time flag end"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_MEDIA_CONFIG_CHANGE,"media config change"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REMAIN_ARTICLE,"remain article"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_PEOPLE_GATHER,"people gather"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ENTER_AREA,"enter area"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_LEAVE_AREA,"leave area"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ARTICLE_MOVE,"article move"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_MOVE_DETECT_RECOVER,"move detect recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VIDEO_LOST_RECOVER,"video lost recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VIDEO_TAMPER_RECOVER,"video tamper recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_INPUT_SWITCH_RECOVER,"input switch recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_TEMPERATURE_RECOVER,"temperature recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_AUDIO_DETECT_RECOVER,"audio detect recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_ABNORMAL_RECOVER,"disk abnormal recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_STORAGE_WILL_FULL_RECOVER,"disk storage will full recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_STORAGE_IS_FULL_RECOVER,"disk storage is full recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_DISABLED_RECOVER,"disk raid disabled recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_DEGRADED_RECOVER,"disk raid degraded recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STOR_ERR_RECOVER,"storage error recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STOR_DISOBEY_PLAN_RECOVER,"storage disobey plan recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_IMAGE_BLURRY_RECOVER,"image blurry recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_TRACK_RECOVER,"smart track recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_IP_CONFLICT,"ip conflict"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_IP_CONFLICT_CLEARED,"ip conflict cleared"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_READ_ERROR_RATE,"smart read error rate"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_SPIN_UP_TIME,"smart spin up time"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_START_STOP_COUNT,"smart start stop count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_REALLOCATED_SECTOR_COUNT,"smart reallocated sector count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_SEEK_ERROR_RATE,"smart seek error rate"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_POWER_ON_HOURS,"smart power on hours"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_SPIN_RETRY_COUNT,"smart spin retry count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_CALIBRATION_RETRY_COUNT,"smart calibration replay count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_POWER_CYCLE_COUNT,"smart power cycle count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_POWEROFF_RETRACT_COUNT,"smart power off retract count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_LOAD_CYCLE_COUNT,"smart load cycle count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_TEMPERATURE_CELSIUS,"smart temperature celsius"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_REALLOCATED_EVENT_COUNT,"smart reallocated event count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_CURRENT_PENDING_SECTOR,"smart current pending sector"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_OFFLINE_UNCORRECTABLE,"smart offline uncorrectable"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_UDMA_CRC_ERROR_COUNT,"smart udma crc error count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_MULTI_ZONE_ERROR_RATE,"smart multi zone error rate")

};

public static int mAlarmcnt = 0;

public static List<String> mAlarmlist = new ArrayList<String>();

public static List<String> getData(int offset, int maxnumber)

{

return mAlarmlist;

}

/*

* Callback function to receive alarm information

* @param [IN] iUserID              	User login ID

* @param [IN] iChannelID           	Channel ID

* @param [IN] dwIndex                  index

* @param [IN] iAlarmType          	Alarm type

* @param [IN] tAlarmTime           	Alarm time

* @param [IN] strName              	Alarm source name

* @param [IN] iBufLen             	Length of structure for alarm information

* @note*/

@SuppressLint("SimpleDateFormat")

public void alarmCallBack(long iUserID, int iAlarmSrcType, int iChannelID, int dwIndex, int iAlarmType, int tAlarmTime, String strName) {

SimpleDateFormat sdf = new SimpleDateFormat("yyyy-MM-dd HH:mm:ss");

long timeLong = Long.valueOf(tAlarmTime);

String  strTime =  sdf.format(new Date(timeLong * 1000L));

String strAlarmInfo = "";

for (MapAlarmType st : alarmTypeMap) {

if(iAlarmType == st.dwAlarmType)

{

strAlarmInfo = st.strAlarmType;

}

}

if(strAlarmInfo == "")

{

return;

}

mAlarmlist.add(0, strTime + " \r\n ChannelID : " + iChannelID  + " : " + strAlarmInfo + " " + strName);

mAlarmcnt++;

System.out.println("Android Report alarm info , iAlarmSrcType:" +  iAlarmSrcType + ", Time :" +  strTime + " UserID : " + iUserID + " , ChannelID : " + iChannelID +  " Index: " + dwIndex + ",AlarmType : " + strAlarmInfo);

}

}

## AlarmMessCallBackV30

/* Alarm callback */

public class AlarmMessCallBackV30 {

MapAlarmType[] alarmTypeMap = {

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_MOVE_DETECT,"move detect"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_MOVE_DETECT_RECOVER,"Motion detection alarm recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VIDEO_LOST,"video lost"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VIDEO_LOST_RECOVER,"Video loss alarm recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VIDEO_TAMPER_DETECT,"video tamper detect"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VIDEO_TAMPER_RECOVER,"Tampering detection alarm recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_INPUT_SWITCH,"input switch"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_INPUT_SWITCH_RECOVER,"Boolean input alarm recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_TEMPERATURE_HIGH,"temperature high"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_TEMPERATURE_LOW,"temperature low"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_TEMPERATURE_RECOVER,"Temperature alarm recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_AUDIO_DETECT,"audio detect"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_AUDIO_DETECT_RECOVER,"Audio detection alarm recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SERVER_FAULT,"server failure"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SERVER_NORMAL,"server failure recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_ONLINE,"device online"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_OFFLINE,"device offline"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_REBOOT,"device reboot"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_SERVICE_REBOOT,"device service reboot"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_CHL_ONLINE,"device chl online"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_CHL_OFFLINE,"device chl offline"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REPORT_DEV_DELETE_CHL,"device delete chl"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_NET_FAILED,"network failed"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_NET_TIMEOUT,"network timeout"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SHAKE_FAILED,"shake failed"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STREAMNUM_FULL,"stream num full"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STREAM_THIRDSTOP,"stream third stop"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_FILE_END,"File ended"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_RTMP_CONNECT_FAIL,"RTMP connect fail"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_RTMP_INIT_FAIL,"RTMP init fail"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STREAM_DOWNLOAD_OVER,"vms gb stream download finished"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_ERROR,"device disk error"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_DISK_ERROR,"system disk error"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_ONLINE,"device disk online"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_DISK_ONLINE,"system disk online"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_OFFLINE,"device disk offline"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_DISK_OFFLINE,"system disk offline"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_ABNORMAL,"disk abnormal"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_ABNORMAL_RECOVER,"disk abnormal recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_STORAGE_WILL_FULL,"disk storage will pull"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_STORAGE_WILL_FULL_RECOVER,"disk storage will full recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_STORAGE_IS_FULL,"device disk storage is full"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_DISK_STORAGE_IS_FULL,"system disk storage is full"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_STORAGE_IS_FULL_RECOVER,"disk storage is full recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_DISABLED_RECOVER,"disk raid disabled recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_DEGRADED,"device disk raid degraded"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_DISK_RAID_DEGRADED,"system disk raid degraded"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_DISABLED,"device disk raid disabled"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_DISK_RAID_DISABLED,"system raid disabled"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_DEGRADED_RECOVER,"disk raid degraded recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STOR_GO_FULL,"device storage full"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_STOR_GO_FULL,"system storage full"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ARRAY_NORMAL,"device disk raid normal"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_ARRAY_NORMAL,"system disk raid normal"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DISK_RAID_RECOVERED,"disk raid recovered"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STOR_ERR,"device store error"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_STOR_ERR,"system storage error recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STOR_ERR_RECOVER,"storage error recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STOR_DISOBEY_PLAN,"store error "),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_STOR_DISOBEY_PLAN_RECOVER,"storage disobey plan recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_BANDWITH_CHANGE,"device export bandwidth change"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VIDEOENCODER_CHANGE,"device stream config change"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_IP_CONFLICT,"ip conflict"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_IP_CONFLICT_CLEARED,"ip conflict cleared"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_NET_OFF,"network disconnect"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_NET_RESUME_ON,"network disconnect recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ILLEGAL_ACCESS,"device illegal access"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SYS_ILLEGAL_ACCESS,"system illegal access"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_LINE_CROSS,"line cross"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_OBJECTS_INSIDE,"inside"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_FACE_RECOGNIZE,"face recognize"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_IMAGE_BLURRY,"image blurry"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SCENE_CHANGE,"scene change"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_TRACK,"smart track"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_LOITERING_DETECTOR,"loitering detector"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_BANDWIDTH_CHANGE,"Bandwidth change"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ALLTIME_FLAG_END,"all time flag end"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_MEDIA_CONFIG_CHANGE,"media config change"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_REMAIN_ARTICLE,"remain article"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_PEOPLE_GATHER,"people gather"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ENTER_AREA,"enter area"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_LEAVE_AREA,"leave area"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ARTICLE_MOVE,"article move"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_FACE_MATCH_LIST,"face recognize blacklist"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_FACE_MATCH_LIST_RECOVER,"face recognize blacklist recovery"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_FACE_MISMATCH_LIST,"face recognize unmatch"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_FACE_MISMATCH_LIST_RECOVER,"face recognize unmatch recovery"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_VEHICLE_MATCH_LIST,"vehicle recognize match"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_VEHICLE_MATCH_LIST_RECOVER,"vehicle recognize match recovery"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_VEHICLE_MISMATCH_LIST,"vehicle recognize unmatch"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_VEHICLE_MISMATCH_LIST_RECOVER,"vehicle recognize unmatch recovery"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_IMAGE_BLURRY_RECOVER,"image blurry recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_TRACK_RECOVER,"smart track recover"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_READ_ERROR_RATE,"smart read error rate"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_SPIN_UP_TIME,"smart spin up time"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_START_STOP_COUNT,"smart start stop count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_REALLOCATED_SECTOR_COUNT,"smart reallocated sector count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_SEEK_ERROR_RATE,"smart seek error rate"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_POWER_ON_HOURS,"smart power on hours"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_SPIN_RETRY_COUNT,"smart spin retry count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_CALIBRATION_RETRY_COUNT,"smart calibration replay count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_POWER_CYCLE_COUNT,"smart power cycle count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_POWEROFF_RETRACT_COUNT,"smart power off retract count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_LOAD_CYCLE_COUNT,"smart load cycle count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_TEMPERATURE_CELSIUS,"smart temperature celsius"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_REALLOCATED_EVENT_COUNT,"smart reallocated event count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_CURRENT_PENDING_SECTOR,"smart current pending sector"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_OFFLINE_UNCORRECTABLE,"smart offline uncorrectable"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_UDMA_CRC_ERROR_COUNT,"smart udma crc error count"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_SMART_MULTI_ZONE_ERROR_RATE,"smart multi zone error rate"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_RESOLUTION_CHANGE,"resolution change"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_MANUAL,"manual"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_ALARMHOST_COMMON,"alarm host common"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_DOORHOST_COMMON,"door host common"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_FACE_NOT_MATCH,"face not match"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_FACE_MATCH_SUCCEED,"face match succeed"),

new MapAlarmType(NetDEVSDK.NETDEV_ALARM_TYPE_E.NETDEV_ALARM_VEHICLE_BLACK_LIST,"vehicle blacklist"),

};

public static List<String> mAlarmlist = new ArrayList<String>();

public static int mAlarmcnt = 0;

public void AlarmMessCallBackV30(long lpUserID, NETDEV_REPORT_INFO_S stReportInfo, long lpBuf, int dwBufLen, long lpUserData)

{

SimpleDateFormat sdf = new SimpleDateFormat("yyyy-MM-dd HH:mm:ss");

long timeLong = Long.valueOf(stReportInfo.stAlarmInfo.tAlarmTimeStamp);

String  strTime =  sdf.format(new Date(timeLong * 1000L));

String strAlarmInfo = "";

for (MapAlarmType st : alarmTypeMap) {

if(stReportInfo.stAlarmInfo.dwAlarmType == st.dwAlarmType)

{

strAlarmInfo = st.strAlarmType;

}

}

if(strAlarmInfo == "")

{

return;

}

mAlarmlist.add(0, strTime + " \r\n ChannelID : " + stReportInfo.stAlarmInfo.dwChannelID  + " : " + strAlarmInfo + " " + stReportInfo.stAlarmInfo.szAlarmSrc);

//CAlarmReport.updateAlarmInfoPage();

mAlarmcnt++;

System.out.println("Android Report alarm info , iAlarmSrcType:" +  stReportInfo.stAlarmInfo.dwAlarmType + ", Time :" +  strTime + " UserID : " + lpUserID + " , ChannelID : " + stReportInfo.stAlarmInfo.dwChannelID  + ",AlarmType : " + strAlarmInfo);

}

}

## ExceptionCallBack_PF

/* Callback function for receiving exception and reconnection messages */

public class ExceptionCallBack_PF {

/*

* Callback function to receive alarm information

* @param [IN] iUserID              	User login ID

* @param [IN] iChannelID           	Channel ID

* @param [IN] iAlarmType          	Alarm type

* @param [IN] tAlarmTime           	Alarm time

* @param [IN] strName              Alarm source name

* @param [IN] iBufLen             	Length of structure for alarm information

* @note*/

public void exceptionCallBack(long iUserID, int iExceptionType, long iExceptionHandle) {

if(iUserID == NetDEVSDK.lpUserID)

{

if(0x8001 == iExceptionType) {

System.out.println("Android Report exception info , Exception Type :" + iExceptionType);

AlarmMessCallBackV30 pfAlarmCallBackV30 = new AlarmMessCallBackV30();

NetDEVSDK.NETDEV_Android_SetAlarmCallBack_V30(NetDEVSDK.lpUserID, pfAlarmCallBackV30, 0);

}

}

}

}

## NETDEV_DISCOVERY_DEVINFO_S

public class NETDEV_DISCOVERY_DEVINFO_S implements Serializable {

private static final long serialVersionUID = 1L;

public String    szDevAddr;        /* Device address */

public String    szDevModule;      /* Device model */

public String    szDevSerailNum;    /* Device serial number */

public String    szDevMac;         /* Device MAC address */

public String    szDevName;        /* Device name */

public String    szDevVersion;       /* Device version */

public int      enDevType; /* Device type NETDEV_DEVICE_TYPE_E */

public int       dwDevPort;       /* Device port number */

public String    szManuFacturer;   /* Device manufacturer */

public String    szActiveCode;     /* Activation code */

public String    szCloudUserName;  /* Cloud username */

}

## NETDEV_DEV_ADDR_INFO_S

public class NETDEV_DEV_ADDR_INFO_S implements Serializable {

private static final long serialVersionUID = 1L;

public String    szUserName;       /* Username */

public String    szPassword;        /* Password */

public String    szIPv4Address;      /* IP address of IPv4 */

public String    szIPv4GateWay;     /* Gateway of IPv4 */

public String    szIPv4SubnetMask;   /* Subnet mask of IPv4 */

public String    szDevSerailNum;     /* Device serial number */

public String    szDevMac;          /* Device MAC address */

}